Documentation for Hiterm

INTRODUCTION

HiTerm is a VT-100 terminal emulation program which runs on IBM PC/XT/AT

(or compatible) computers and the NEC PC-9800 family of PCs. HiTerm

allows the PC to act like a VT-100 terminal and allows you to communicate

with, and exchange files with, other computers or machines (such as

the UniSite Universal Programmer, 2900, or 3900 Programming Systems).

Features

~~~~~~~~

HiTerm is easy to use and provides the following capabilities:

Supports the programmer's High Speed Download feature (115.2K

baud), which can greatly reduce download times for large files if

HiTerm is run on a 286, 386, or 486 PC.

Allows you to control a Data I/O programmer from any IBM or

NEC PC or compatible.

Allows you to perform file transfer operations between the programmer

and the host PC without leaving the programmer's menus.

Supports 8-bit data transfers, which permit users to transfer

binary files quickly and efficiently using any of the binary data

formats supported by a Data I/O programmer.

Supports software handshaking (Xon/Xoff) for flow control.

HiTerm has two operating modes, General (G) and Programmer (P).

General mode allows HiTerm to perform like a VT-100 terminal.  Use

General mode when you wish to transfer files between a host computer

and a PC.

Programmer mode, in addition to its terminal emulation capability,

automatically opens and closes files when transferring data files

between a PC and a UniSite, 2900, or 3900 programmer. This mode also

supports high speed download operations. Choose programmer mode when

using a UniSite, 2900, or 3900 programmer.

Note: All references to programmer refer to UniSite, the 2900,

or 3900 Programming Systems. Do not use General mode to control UniSite,

the 2900, or 3900 programmer; use Programmer mode, which is designed

to be used with these programmers.

Contents of HiTerm Disk

~~~~~~~~~~~~~~~~~~~~~~~

program.bat     Batch file used to invoke HiTerm in Programmer

mode

prg4800.cfg     Configuration file (Programmer mode,

4800 baud)

prg9600.cfg     Configuration file (Programmer mode, 9600

baud)

prg19200.cfg    Configuration file (Programmer mode,

19200 baud)

general.bat     Batch file used to invoke HiTerm in the

General mode

gen4800.cfg     Configuration file (General mode,

4800 baud)

gen9600.cfg     Configuration file (General mode, 9600

baud)

gen19200.cfg    Configuration file (General mode, 19200

baud)

hiterm.exe      Terminal emulator program

manual.doc      Documentation for HiTerm

sample.dat      Practice file for transferring files between

the PC and the programmer and for editing files. Note that

this practice file contains intentional typing errors which

you can correct with the programmer editor.

------------------------------------------------------------------------------

INSTALLATION

HiTerm is provided on a 360K double-sided, double density 5-1/4 inch

disk in IBM format. You can run HiTerm from a floppy or hard disk.

On a Floppy Drive

~~~~~~~~~~~~~~~~~

Follow the steps below to make a backup copy of HiTerm.

1. Make a working copy of the HiTerm disk by using the DOS

COPY *.* command. If you have questions, consult your DOS manual.

2. Save the original disk as a master.

3. Proceed to Chapter 3, "Operation."

On a Hard Drive

~~~~~~~~~~~~~~~

Follow the steps below to install HiTerm on your hard disk.

1. Copy all the files from the HiTerm disk to the hard disk

in any subdirectory you choose.

2. Modify the autoexec.bat file in the root directory

of the hard disk so the PATH points to the subdirectory where the

HiTerm files reside (if it doesn't already point there). For more

information regarding the path command, refer to your DOS manual.

Modify program.bat File

~~~~~~~~~~~~~~~~~~~~~~~

3a. If you are using HiTerm in Programmer mode, edit the

program.bat configuration file to reflect the location of the

configuration files. Edit the two lines that invoke HiTerm so they

point to the drive and subdirectory containing the HiTerm files. Refer

to the following example.

Original program.bat File

~~~~~~~~~~~~~~~~~~~~~~~~~

echo off

Rem: HITERM will use the configuration filename from command line if present.

If not (%1) == () HITERM %1

Rem: HITERM will use PRG9600.CFG if no configuration file is specified.

If (%1) == () HITERM PRG9600.CFG

Modified program.bat File

~~~~~~~~~~~~~~~~~~~~~~~~~

echo off

Rem: HITERM will use the configuration filename from command line if present.

If not (%1) == () HITERM C:\UTIL\%1

Rem: HITERM will use PRG9600.CFG if no configuration file is specified.

If (%1) == () HITERM C:\UTIL\PRG9600.CFG

Modify general.bat File

~~~~~~~~~~~~~~~~~~~~~~~

3b. If you are using HiTerm in General mode, edit the

general.bat file to reflect the location of the configuration

files. Edit the two lines that invoke HiTerm so they point to the

drive and subdirectory containing the HiTerm files. Refer to the following

example. (This step is not necessary if you will not be using HiTerm

in General Mode.)

Original general.bat File

~~~~~~~~~~~~~~~~~~~~~~~~~

echo off

Rem: HITERM will use the configuration filename from command line if present.

If not (%1) == () HITERM = C:\UTIL\%1

Rem: HITERM will use GEN9600.CFG if no configuration file is specified.

If (%1) == () HITERM GEN9600.CFG

Modified general.bat File

~~~~~~~~~~~~~~~~~~~~~~~~~

echo off

Rem: HITERM will use the configuration filename from command line if present.

If not (%1) == () HITERM C:\UTIL\%1

Rem: HITERM will use GEN9600.CFG if no configuration file is specified.

If (%1) == () HITERM C:\UTIL\GEN9600.CFG

4. Installation is now complete. Reboot your system.

------------------------------------------------------------------------------

OPERATION

You can run HiTerm in Programmer (P) mode or General (G) mode. Use

Programmer mode if you are using HiTerm with a UniSite, 2900, or 3900

programmer. Use General mode when you are using a PC to communicate

with other Data I/O programmers or with another computer.

Running HiTerm in Programmer Mode

From a Floppy Disk

~~~~~~~~~~~~~~~~~~

To run HiTerm from drive A, place the HiTerm disk in drive

A and enter:

A: RETURN

Next enter:

program RETURN

HiTerm will be invoked using the prg9600.cfg configuration file.

If you want to use a different configuration file, enter the configuration

filename following the word program. If you enter

program prg19200.cfg RETURN

HiTerm will be invoked using the prg19200.cfg file.

If you want to execute HiTerm from a drive other than the one you

are on, or from a subdirectory, you must set the DOS path variable

to point to where HiTerm resides. You must also modify the program.bat

file to point to the location of the configuration files. Refer

to Chapter 2 and the instructions for installing HiTerm on a hard

disk for more information on editing the program.bat file.

From a Hard Disk

~~~~~~~~~~~~~~~~

After you have installed the HiTerm files on your hard disk,

simply enter:

program RETURN

to start HiTerm in Programmer mode. When you start HiTerm, it

reads a configuration file to set the operating mode (General or Programmer)

and to set the various communication port parameters, such as baud

rate, parity, and com port. To run HiTerm at a different baud rate,

enter the appropriate configuration filename on the command line following

the Program command. For example:

If you enter program prg19200.cfg, HiTerm will be invoked using the

prg19200.cfg configuration file, and HiTerm will run at 19200 baud on Com1.

If a configuration file is not specified after you enter program,

the program.bat batch file will use the prg9600.cfg file

by default, and HiTerm will run at 9600 baud on Com1.

Running HiTerm in General Mode

This mode is not recommended if you are using a UniSite, 2900, or

3900 programmer. Use Programmer mode instead because it provides more

capabilities, such as High Speed Download.

From a Floppy Disk

~~~~~~~~~~~~~~~~~~

To run HiTerm from drive A, place the HiTerm disk in drive

A and enter

A:RETURN

Then enter

general RETURN

HiTerm will be invoked using the gen9600.cfg configuration

file. If you want to use a different configuration file, enter the

configuration filename following the word general. If you enter

general gen19200.cfg RETURN

HiTerm will be invoked using the prg19200.cfg file.

If you want to execute HiTerm from a drive other than the one you

are on, or from a subdirectory, you must set the DOS path variable

to point to where HiTerm resides. You must also modify the general.bat

file so that it points to the location of the configuration files.

Refer to Chapter 2 and the instructions for installing HiTerm on a

hard disk for more information on editing the general.bat file.

From a Hard Disk

~~~~~~~~~~~~~~~~

After you have installed the HiTerm files on your hard disk,

simply enter

general RETURN

to start HiTerm in General mode. When you start HiTerm, it reads

a configuration file to set the operating mode (General or Programmer)

and to set the various communication port parameters (baud rate, parity,

com port, etc.). To run HiTerm at a different baud rate, enter the

appropriate configuration filename on the command line following the

word general. For example:

Entering general gen19200.cfg will invoke HiTerm using

the gen19200.cfg configuration file, which will cause HiTerm

to run at 19200 baud on Com1.

If a configuration file is not specified after the word "general,"

the general.bat batch file will use the gen9600.cfg file

by default which will cause HiTerm to run at 9600 baud on Com1.

HiTerm Configuration File

~~~~~~~~~~~~~~~~~~~~~~~~~

HiTerm uses a configuration file to set various system parameters

such as operating mode (General or Programmer), PC type (IBM or NEC),

and communication parameters (such as baud rate). If HiTerm is not

able to read the configuration file, or if there is an error in the

file, a message is printed indicating the error. You will then be

prompted to abort HiTerm or to continue. If you elect to continue,

HiTerm will use default values for the items specified in the configuration

file. See the following page for a list of defaults.

Several configuration files are included on the HiTerm disk; create

any additional configuration files you desire. When you create your

own configuration file, be sure each line conforms to the specifications

shown below. Configuration files allow you to set HiTerm parameters

specific to your setup. The first field of each line in the configuration

file specifies a particular setup parameter. The order of the lines

in the file must be as follows:

First Line - Mode.

Specify either General (G) or Programmer (P). You can use upper- or

lowercase; only the first character of the word is significant.

Second Line - Baud Rate.

The complete number is required. For example, use 9600, not 96.

Third Line - Parity.

Specify None, Odd, or Even (N, O, or E). Use upper- or lowercase;

only the first character of the word is significant.

Fourth Line - Data Bits.

Specify 7 or 8.

Fifth Line - Stop Bits.

Specify 1 or 2.

Sixth Line - Com Port.

Specify 1 or 2.

Seventh Line - PC Type (I = IBM type, N = NEC type, A

= automatic detection)

This parameter specifies which type of PC you are using (IBM or NEC

type). The software can automatically detect the type of PC you are

using and configure itself accordingly if this parameter is set to

A (default). You can bypass the automatic selection feature by specifying

either I for IBM and compatibles, or N for NEC type PCs (PC-9800 family).

Example Configuration File

~~~~~~~~~~~~~~~~~~~~~~~~~~

An example of a configuration file is shown below.

Parameter Field     Comment Field (Not Used By HiTerm)

P                   Configuration file for HiTerm. First line is Mode (G or P).

9600                Second line is Baud Rate.

N                   Third line is Parity (None, Odd, Even).

8                   Fourth line is number of Data Bits per character (7 or 8).

1                   Fifth line is number of Stop Bits (1 or 2).

1                   Sixth line is the Com Port number (1 or 2).

A                   Seventh line is Computer Type (I = IBM or compatible,

N = NEC, A = automatic detection).

Note: Comments can follow the first word (or character) as long as they are

separated from the word by a space and are contained on that one line. The

line must end with a line feed. (A RETURN is optional.)

Baud Rates

HiTerm supports the following baud rates:

50      600      3600

75      1200     4800

110     1800     7200

150     2000     9600

300     2400     19200

HiTerm Default Settings

The following are the factory default settings and are used

if no configuration file is present or if HiTerm cannot read the specified

configuration file.

Parameter           Factory Default Setting

Mode                P (Programmer)

Baud Rate           9600

Parity              N (None)

Data Bits           8

Stop Bits           1

Com Port            1

PC type             A (Automatic Detection of PC Type)

Transferring Files with HiTerm (Programmer Mode)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

A special host command, transfer, is used by the programmer

to communicate with HiTerm. Follow the steps below to either upload

or download files. The files will be opened and closed automatically

on the PC at the appropriate times.

Upload

~~~~~~

1. On the More commands/Transfer data/Upload menu of

the programmer, make sure the Destination field reflects the correct

programmer serial port, Terminal (T) or Remote (R).

2. In the Upload host command field at the bottom of the

screen, enter transfer filename RETURN

or alternately, tr filename RETURN

To enter the parameter, enter another

RETURN to perform the upload.

Download

~~~~~~~~

1. On the More commands/Transfer data/Download menu

of the programmer, make sure the Source field reflects the correct

Programmer serial port, Terminal (T) or Remote (R).

2. In the Download host command field at the bottom of the

screen, enter

transfer filename RETURN

or, alternately, tr filename RETURN

At this point the parameter is entered. Enter another

RETURN

to perform the download.

If you are using General (G) mode, refer to the HiTerm Help screen

for the key assignments regarding sending files to or from the PC.

High Speed Download is not available in General mode.

High Speed Mode

~~~~~~~~~~~~~~~

General Information

HiTerm supports high speed downloads at 115.2K baud. This

can greatly reduce download times for large files if HiTerm is run

on a 286, 386, or 486 PC. The PC translates the data and then sends

it to the Com port.

The download speed is affected primarily by two factors: speed of

the processor in the PC and format of the data in the file to be downloaded.

Since the PC will be translating the data, a faster processor will

translate data and supply data to the Com port at a faster rate. The

file format affects the speed because some formats require less time

to translate (such as binary formats).

When a high speed download occurs, data will be transferred at

115.2K baud regardless of the type of processor involved. However,

slower processors may not supply the data to the Com port at the rate

required to maintain a continuous flow of data to the programmer. Consequently,

the download time will be slower even though the baud rate is 115.2K

baud. To benefit from the high speed download feature, run HiTerm

on a PC at least as powerful as a 6 MHz 286.

During a high speed download, the PC translates the data in the user's

file from its original format into an internal format required by

the programmer. The formats currently supported by HiTerm for high

speed downloads are listed below.

Format Name                     Format Number

Binary                               10

Intel Intellec 8/MDS                 83

Intel MCS-86 Hex Object              88

Intel Hex-32                         99

JEDEC (Full)                         91

JEDEC (Kernel)                       92

Motorola EXORcisor                   82

Motorola EXORmax                     87

Motorola 32-bit (S3 record)          95

Performing a High Speed Download

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Use the following procedure to perform a high speed download.

1. From the More commands/Configure system/Edit/Serial I/O parameters screen,

ensure that the baud rate for the Remote port matches that of the Terminal

port.

2. Go to the More commands/Configure system/Edit/Communication

parameters screen.

3. Set the User Menu Port parameter to R (Remote). If it already indicates

R, proceed to Step 6.

4. Enter RETURN in response to the following message:

Hit return to switch user menu port, ^Z to abort.

5. Move the cable currently on the Terminal port of the programmer

to the Remote port.

6. Set the high speed download parameter to Y.

7. Go to the More Commands/Transfer/Download screen.

8. Set the Source field to R (Remote).

9. Select the appropriate I/O translation format. You must

use one of the formats currently supported by HiTerm to perform a

high speed download. Otherwise, the transfer occurs at the normal

baud rate.

10. To initiate the transfer, in the Download Host Command

field, enter transfer filename RETURN or, alternately,

tr filename RETURN

When you start the high speed download, the following message is displayed

on the programmer screen:

Transferring data in High Speed Mode

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When this message appears, the programmer and HiTerm will automatically

switch baud rates to 115.2K baud and perform the data transfer. When

the transfer is complete, the baud rates are restored to their original

rates and a message is displayed indicating the operation is complete.

If the format number you have selected is not one of those supported

for high speed download, the message

Transferring data

is displayed and the transfer occurs at the normal baud rate.

CAUTION: Do not remove or insert device modules during high speed download

operations.

------------------------------------------------------------------------------

COMMANDS AND FUNCTIONS

The functions accessible from the Programmer mode Help menus are described

below. Since the NEC computers don't have an ALT key, you invoke the HiTerm

functions using the CTRL key with a function key. See Chapter 5 for more NEC-

specific operation information.

----------Command------------

IBM PC                 NEC             Function

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~         ~~~~~~~~

Ctrl-R                 Ctrl-R          Repaints the programmer screen.

Alt-F1                 Ctrl-F1         Terminates the HiTerm program, returns

to DOS, and closes any open files.

Alt-F2                 Ctrl-F2         Switches between Text mode and Binary

mode for transferring files to the Com

port.

In Text mode the following characters

are stripped from the file during

transfer: Control-Z, Control-C, Line-

Feed, and Nulls. When you transfer a

file from the PC to a host while in

Text mode, an End-Of-File (EOF)

character is sent after the data to

indicate the end of the file.

In Binary mode, no characters are

stripped and no EOF character is sent

following the end of the data. Binary

mode is the default and is the

recommended mode for operation with

your programmer.

CAUTION: Do not use Text mode to transmit files in JEDEC format or any

of your programmer's binary formats, since filtering of control characters

may result.

----------Command------------

IBM PC                 NEC             Function

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~         ~~~~~~~~

Alt-F3                 Ctrl-F3         Displays the HiTerm help screen.

Alt-F4                 Ctrl-F4         Displays the current End-Of-File (EOF)

character for text mode file

transfers. To change the EOF character,

enter the hexadecimal value of the

desired  control character (ASCII 01

through 1F) and press RETURN to select

the new character. The default setting

is Control-Z (1A Hex). This parameter

applies only to text mode. If Text mode

is used, the EOF character must match

the character selected on the

programmer's Communications Parameters

screen.

Note: HiTerm operating in Programmer (P) mode automatically closes the file

after Upload and Download operations without needing an EOF character.

----------Command------------

IBM PC                 NEC             Function

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~          ~~~~~~~~

Alt-F5                 Ctrl-F5         Allows you to change directories from

within HiTerm. Enter the path for the

directory you want to change to. You

may also change to a different drive by

entering the drive letter followed by a

colon (a:\util).

Alt-F6                 Ctrl-F6         Displays the directory for the drive

and path specified.

Ctrl-Break             Ctrl-F10        Sends a Break signal to the Com Port.

The status line at the bottom of the screen displays the current settings

of various HiTerm parameters and also indicates how to access the

HiTerm help screen: Alt-F3.

The fields on the status line are described below.

[Caps]          The Caps-Lock key is enabled.

[Num]           The Num-Lock key is in the Number mode.

[Text]          The file-transfer mode is set to Text mode.

[Bin]           The file-transfer mode is set to Binary mode.

[EOF=XX]        Indicates the EOF character (only used for

file transfers in text mode).

------------------------------------------------------------------------------

MESSAGES

HiTerm messages appear at the bottom of the screen.

Can't change directory

HiTerm was not able to change to the specified directory. Make sure the

directory you entered is correct and try again.

Chars lost

HiTerm encountered an error when attempting to close a file during a download

operation. Reattempt the download operation.

CTS pin for com port not on

This message applies only to NEC PCs. HiTerm was unable to send data out of

the Com port because the Clear To Send (CTS) pin (pin 5) on the port was not

being asserted HIGH by the programmer. Either the programmer is not powered

on, or pin 5 is not connected in the interface cable. The programmer asserts

the required level on this line, but the cable must have pin 5 connected for

communication to occur. The cable supplied with the programmer meets this

requirement. If pin 5 is not connected in the cable you are using, connect pin

5 (CTS) to pin 20 (DTR) on the PC end of the cable to establish communication.

Data overrun on com port

The UART on the PC received another character before the previous one was read

by the processor, causing data to be lost. Try using a slower baud rate or

eliminate any background programs currently running on the PC.

Default parameters used

HiTerm was not able to use the information in the configuration file. The

default values for the parameters will be used.

EOF not used in Bin mode

You attempted to set the EOF character while in Binary file transfer mode.

Since the EOF character is used only for Text mode, change the file transfer

mode to Text to use the EOF character.

Error reading line number x in configuration file: "path/filename"

HiTerm encountered an error while reading the line specified. Check the

configuration file to ensure it is not corrupted.

File I/O error

HiTerm encountered an error during a file access operation. If this occurred

while attempting to send a file from the PC to the Com port, verify that the

file actually exists. If this error occurred while transferring data to a

file, ensure that enough space exists on the disk to accommodate the data.

File(s) not found

HiTerm was unable to find the file specified. Make sure that the filename

entered is correct and try again.

Framing error on com port

The character received at the Com port did not contain a valid Stop Bit. Make

sure the baud rates of the PC and the programmer match. If the problem still

exists, try using a slower baud rate.

Illegal baud rate specified in configuration file "path/filename"

HiTerm encountered an illegal baud rate in the configuration file. Ensure that

the value specified is one of those listed in Chapter 3 as a legal baud rate.

If you are using an NEC PC, also refer to Chapter 5.

Illegal com port specified in configuration file "path/filename".

Com port 2 not supported for NEC computers.

HiTerm supports either Com1 or Com2 on IBM type PCs. For NEC type PCs, only

Com1 is supported.

Illegal parameter in configuration file: "path/filename" line

number: x

HiTerm detected a syntax error on the line number specified in the

configuration file. Check the contents of the configuration file used

(prg9600.cfg, if not specified on the command line) to ensure there are

no syntax errors. If you choose to continue, HiTerm will use the default

parameters.

Not enough memory for HiTerm

There is not enough free memory available in the PC. HiTerm requires a minimum

of 128K bytes of RAM. Remove other programs from memory or add more RAM.

Parity error on com port

The parity of the data received did not correspond with the parity setting of

the UART in the PC. Make sure the parity settings in the PC and the programmer

are the same.

The configuration file does not contain all of the required parameters.

This may be caused by using an old version of the configuration file.

One or more parameters are missing from the configuration file. Verify that

the configuration files you are using contain all of the parameters required

by HiTerm. A new parameter (PC type) was added to the configuration file in

HiTerm Version 3.10. If you are using configuration files that were used with

previous versions of HiTerm, you should update them to contain any new

parameters.

Transfer Error

HiTerm did not close the file on the PC automatically. Make sure the HiTerm

mode is set to Programmer (P). If the mode is correct, try using the pacing

delay feature on the programmer, or a slower baud rate.

Unable to close configuration file "path/filename"

HiTerm encountered a file error when attempting to close the configuration

file specified. Check the configuration file to ensure it is not corrupted.

Unable to open configuration file "path/filename": (No such file or directory)

HiTerm couldn't find the configuration file indicated. Make sure the

configuration file entered is correct and try again.

------------------------------------------------------------------------------

TROUBLESHOOTING

When you are using HiTerm in Programmer mode and you cannot invoke

HiTerm or you are having difficulty with the transfer command, check

the following.

1. Invoke the HiTerm Help screen  Alt-F3 and make sure the HiTerm operating

mode is set to Programmer (P). The mode is specified in the configuration

file.

2. Make sure the correct syntax is used on the host command line of the

programmer transfer menu when you are using the Transfer command.

3. If the programmer menu is not present or complete on the PC screen, press

Ctrl-R to repaint the Programmer menu.

4. Type program to invoke HiTerm. Do not type the word HiTerm, since

doing this will bypass the program.bat batch file, which is necessary

for invoking the default configuration file if one is not specified on the

command line.

5. Make sure the programmer's baud rate matches the HiTerm baud rate. The

programmer's factory default setting is 9600 baud.

6. Make sure the programmer is connected to the correct Com port on the PC

(default is Com1).

7. If the programmer indicates framing, parity, or overrun errors during high

speed downloads, the communication board in your PC may not be able to

support data transfers at 115.2K baud. Disable the High Speed Download

feature or use a different Com board in your PC.

8. Make sure that the terminal type on the programmer is set to VT-100 (ANSI

3.64). You can check this on the programmer's power-up screen.

------------------------------------------------------------------------------

NEC 9800 OPERATION NOTES

Running HiTerm on the NEC PCs is the same as running it on the IBM

(and compatible) type PCs with the following exceptions.

1. Since the NEC PCs don't have the Alt key, HiTerm operations are invoked

using the Ctrl key with the function keys.

2. You cannot use baud rate 19200 with some of the NEC PCs.

If the frequency of the clock feeding the 8253 timer chip for the

serial port in the PC is 2.4576 MHz, then 9600 baud is the fastest

baud rate available for that PC. If the frequency is 1.9968 MHz, then

HiTerm can be set to operate at 19200 baud. HiTerm will automatically

determine which clock frequency is present and then will check that

the baud rate specified in the configuration file is a legitimate

baud rate for that particular PC. If it is not, an error message appears.

3. The baud rate used for high speed download is also determined

by the clock frequency mentioned previously. If the clock frequency

is 2.4576 MHz, baud rate 9600 will be used for high speed downloads.

If the clock frequency is 1.9968 MHz, baud rate 38400 will be used

for high speed downloads.

Baud rates 9600 and 38400 are the fastest baud rates that

can be used for high speed downloads when operating the NEC PCs with

UniSite, the 2900, or 3900. Even though these baud rates are slower

than the 115.2K baud rate possible with the IBM type PCs during high

speed download, a significant improvement in download times can still

be achieved compared with normal speed downloads. This is because

the data file is translated into a binary format on the PC prior to

the download and consequently fewer bytes need to be transferred to

UniSite, the 2900, or 3900.

4. The interface cable used between the NEC PC and the programmer

must have pin 5 (CTS) present in the cable to communicate. (The cable

supplied with the programmer meets this requirement.) The UART in

the NEC PC requires that pin 5 be asserted HIGH to communicate over

the serial port. The programmer will assert the necessary signal on

pin 5. If pin 5 is not connected in the cable you are using, you can

connect pin 5 to pin 20 (DTR) on the PC end of the cable to establish

communication.