I have decided that most the print spoolers for the IBM PC

are just junk. They dont dont have enough flexibility in operation and few

if any can give you updated status of the buffer. Part of the problem lies

in the basic hardware flaws of the IBM PC. It is impossible to generate a

true interrupt driven spooler for the IBM parallel port. This is because the

ACK signal coming back from the printer is tied direct to the interrupt

controller. Unfortunately the timing of this signal,as defined for standard

centronics interface,is too fast to be acknowledge by the controller! It

is uncanny how one simple flip-flop inserted between the signal line and the

controller could have elimnated the problem! All the clones out there

never bothered to include it either,copying IBM line for line!

As a result,all spoolers have to tag themselves to either the

keyboard interrupt or timer. Some tag to both of them. However some control

of priority is needed,or conflicts may occur. This priont spooler alows

that control. THe spooler is divided into 2 parts.

SPOOL.DEV      : This is the actual device driver for the spooler.

BUFFERP.COM      : This is control program that sends and recieves

: information from the device driver. Allow priority

: or port changing,etc!

A detail of the operation is given below,and how to install the

program. Source is also included for hackers to use. Although I did not

write this program originally,it was given to me for modification. The company

I worked for on this project has since gone under, and I feel that since I

modified it and the company folded, I have full rights to it. I am giving it

away free in the true spirit of public domain. Sort of a thank you to many of

the other fine PD authors who given us programs like Qmodem,EBL,Util, etc! My

thanks go out to these fine authors!

Using SPOOL.DEV:

The print spooler allows up to 64K of ram to preserved for use as a

print spooler. All output to printer device will be routed to the buffer and

printed to the destination during a 'background' operation. THis will allow

the user to continue using his computer for other functions. The printer

devices allowed for this spooler can be as follows:

LPT1,LPT2,LPT3 or COM1,COM2.

Also the device for spooling can be changed at anytime the device is

installed or after! THe user can set how much memory he wishes to reserve by

adding a 'switch' to the command line during installation. Install the device

by editing a file on the boot disk "Config.Sys". THis may be edited with any

editor such as edlin or Wordstar. Options for the driver are allowed and

installed as folows:

DEVICE=SPOOL.DEV  /60 /L1

Placing this line in the Config.sys file is all that is needed. The first

parameter "/60" indicates how much memory is be reserved for the buffer. It is

in allocated in 1K increments. ANy number is allowed from "1" to "63". If no

number is entered,or an incorrect number, 63 will be the default. The second

parameter indicates the default port for the spooler. Allowable parameters are

as follows:

/L1     : Route to LPT1

/L2     : Route to LPT2

/L3     : Route to LPT3

/C1     : FOr COM1

/C2     : For COM2

Any other parameters are not allowed. The default port,if none are

emtered is LPT1. Make sure that the file SPOOL.DEV is also installed on the

boot disk. Remember to restart the pc before spooler is activated!

The second file BUFFERP.COM is a utility program that allows the user

to change ports for spooling,clear the buffer and see how much is in the

buffer. It is menu driven and very self explanatory. It will only work if the

spooler is installed, otherwise will complain! Execute it by simply typing

"BUFFERP", making sure the file is on the default drive or in the DOS Path

specification.

I have found few problems with the spooler. THe priority feature of the

spooler also comes in handy for controlling output speed. It can changed on

the fly withg the BUFFERP command. Also this is the first spooler I have found

that works well with the DOS PRINT command! SHould work with any programs such

as Wordstar or PC-WRITE! Enjoy it!

Craig Derouen

March 5,1985

Addemium for RBBS sysops or Multi-Link Users

There is a few things the RBBS sysop or Multi-Link user should know about

spool.dev  Under RBBS if operator page is on and a user pages, my Epson printer

will cease to log the activity of the BBS, but continues to fill the buffer. I

have only found Re-boot as a solution to date.

If you are using multi-link It will work fine for either paration, howver

attempting to change the printer priority causes a crash. It will allow buffer

clearing however.

If any rbbs sysop figures out how to make page not conflict with spool, please

leave a comment to the sysop or Craig Derouen at Video Advisor ON-LINE at

206/244-1685

Thank you Tim Houser sysop VA RBBS

##############################################################################################################################