NSSTPNEW.DOC

The following is valid ONLY if you are running North Star CP/M Version 1.1.0!!

This is a change to the BIOS which will allow independent setting of the

step time for fast and slow stepping drives.  If you have the ASM file for

this BIOS (NSBIOS22.ASM), the change is to the WAIT: and WAIT1: routines.

The new code follows with run time addresses given in the remarks to labels

for a 63K system.  ADJUST these addresses for other size sytems:

wait:	mvi	d,22H	;(F095) The value put in D here will determine the

;step time for drives that are declared NORMAL stepping

;during the CPMGEN process.  22H works well for Tandon

;100-2 drives.  A MUCH!! larger value will be needed

;if your drives are really SLOW stepping.

push	psw	;Need to save the contents of A

lda	confg	;(F5F8)  Get the CONFG byte to see if the drive is

;FAST stepping, or NORMAL stepping

cma		;The next three bytes make that determination

ana	l

ani	0FH

mov 	b,a	;Need to save this information for some other use

jnz	wait1	;GOTO wait1 if it is a NORMAL stepping drive

mvi	d,0BH	;Else, load the value in D for your drives declared as

;FAST stepping durning CPMGEN.  0BH seems to work OK

;with Tandon 100-4 drives.

wait1:	pop	psw	;(F0A5)Restore the contents of A

wait2:	dcr	d	;(F0A6)Now do the countdown

jnz	wait1

dcr	a

jnz	wait

nop		;Fill the empty space left over

The next statement is the original XX13: equ $+2

If you have the source for the BIOS, you will note that at call to OFTEN0 is

omitted.  However, OFTEN0 is a jump to OFTEN which is only a return in the

running system.  Thus, the call is not needed.  Also, the CNZ WTSF1 was the

original branch for NORMAL stepping drives, but the code above substitutes

for this.

The following will help you patch this code.  Addresses are given for a 63K

running system, the SYSGEN image, and disk locations for patching with DU.

The code begins on the disk at T0;S6; byte 15.  BE SURE to adjust the label

addresses to match your CP/M size.

63 K		SYSGEN		HEX CODE	Disk location

System		IMAGE				T0;S6; byte 15

F095		2C95		16 (MVI D)	15

F096		2C96		22 (NORM STEP)	16

F097		2C97		F5 (PUSH PSW)	17

F098		2C98		3A (LDA)	18

F099		2C99		F8 (Address of  19

F09A		2C9A		F5  CONFG)	1A

F09B		2C9B		2F (CMA)	1B

F09C		2C9C		A5 (ANA L)	1C

F09D		2C9D		E6 (ANI)	1D

FO9E		2C9E		0F		1E

F09F		2C9F		47 (MOV B,A)	1F

F0A0		2CA0		C2 (JNZ)	20

F0A1		2CA1		A5 (Address of	21

F0A2		2CA2		F0  WAIT1)	22

F0A3		2CA3		16 (MVI D)	23

F0A4		2CA4		0B (FAST STEP)  24

F0A5		2CA5		F1 (POP PSW)	25

F0A6		2CA6		15 (DCR D)	26

F0A7		2CA7		C2 (JNZ)	27

F0A8		2CA8		A6 (Address of	28

F0A9		2CA9		F0  WAIT2)	29

F0AA		2CAA		3D (DCR A)	2A

F0AB		2CAB		C2 (JNZ)	2B

F0AC		2CAC		95 (Address of	2C

FOAD		2CAD		F0  WAIT)	2D

F0AE		2CAE		00 (NOP)	2E

end of patch

A word of caution - Tandon 100-2's (quads) seem to slam into the T0 stop if

the delay value is set less than 22H.  Do not allow this to continue as it will

surely damage the T0 microswitch.  100-4's (octals) just quit reading correctly

at delays lower than 0BH.  If you are using "real" slow stepping drives (SA400,

etc.), I suggest setting the delay to FFH to start and then decreasing it until

read errors occur or there are any signs of hitting a stop too hard.  This has

not been tested with SLOW stepping drives and may not work.

As a matter of interest, the original delay for fast stepping drives was 38H,

so this patch gives over a five fold decrease in seek time!!

Thanks to Sigi Kluger for catching a bug to prevent this code from "blowing

its stack".

Happy stepping,

Phil Cary, REDSTICK RCPM

###########################################################################################################################