RPMCON v1.31

============

RPMCon is a utility that allows you to place location constraints for RPMs

in a special "constraints" file.  This allows you to floorplan RPMs more

easily, without having to place RLOC_ORIGINs on the original schematic and

having to recompile the entire design.

The "constraints" file is called an RCF file (for "RPM constraints file")

and has a syntax similar to that of PPR's CST file.  The format for an RCF

constraint is:

place set <rpm_set_name>: clb_r<row>c<col>;

(Any number of spaces may be inserted around all fields.  The colon and

semicolon may be preceded by spaces, but they must be included or the line

will be ignored.)

The CLB location field specifies the location of the upper-left corner of

the RPM referenced by <rpm_set_name>.  <rpm_set_name> is a value unique to

each RPM in a design, and is the value assigned to the symbol attributes

H_SET, HU_SET, and U_SET.

For example, say an XTF file has the following line:

SYM, TOP/EXAMPLE, DFF, SCHNM=FDCE, H_SET=TOP/hset, RLOC=R1C6.FFX, INIT=R

This is a flip-flop labeled TOP/EXAMPLE which belongs to the RPM set called

TOP/hset.  All instances that belong to this RPM set may be constrained by

the following line in the RCF file:

place set top/hset: clb_r4c2;

(The RCF file is case insensitive.)

An instance in TOP/hset with RLOC=R0C0 would thus be placed in CLB_R4C2.

By transposition, TOP/EXAMPLE, would be placed in CLB_R5C8.FFX.

RPMCon works by reading in the RCF constraints, then rewriting the XTF file

with LOC attributes that reflect such transpositions.  For example, the

SYM record above would be rewritten as:

SYM, TOP/EXAMPLE, DFF, SCHNM=FDCE, LOC=CLB_R5C8.FFX, INIT=R

Note that the H_SET attribute has also been eliminated since it has no

meaning without an RLOC.

The format for the rpmcon command is:

rpmcon <design>[.xtf] [confile=<confile>] [outfile=<outfile>[.xtf]]

The input filename must have a .xtf extension.  The two parameters that

RPMCon accepts are:

confile   The name of the RCF file.  Defaults to <design>.rcf.

outfile   The name of the output netlist.  Defaults to rpmcon.xtf.

It must have a .xtf extension, and cannot have the same name

as the input file.

The keywords "confile" and "outfile" are case insensitive (although, on

Unix platforms, their filenames are not).

RPMCon accepts XTF and RCF files in either DOS or Unix format.  The output

format is consistent with that of the platform on which RPMCon is run.

RPMCon will issue warnings if a set listed in the RCF file does not exist

in the XTF netlist, or if a set is listed more than once in the RCF.  RPMCon

will also print out lines in the RCF file that it does not understand.

(Lines can be commented out with "#".)

NOTE: RPMCon does not check the validity of the SYM records in the netlist.

(It assumes that XNFPrep has done its job.)  It also does not check whether

the location constraints it writes are legal for the target device.