First draft: 11/18/94

=====

Enclosed are the library source files for the Xilinx-OrCAD interface.

These source files are for the Xilinx XACT 5.0 Unified Libraries.  The

files enclosed are:

2000src\

xc2000.src      xc2000.lib schematic library source

3000src\

xblox.src       xblox.lib schematic library source

xc3000.src      xc3000.lib schematic library source

4000src\

xblox.src       xblox.lib schematic library source

xc4000.src      xc4000.lib schematic library source

7000src\

xc7000.src      xc7000.lib schematic library source

The library files shipped with XACT 5.0 are in the OrCAD 386+,

32 bit data format.  These are NOT compatible with OrCAD v4.20.

The source files (.SRC) provided here can be recompiled for use with

OrCAD 4.20.

Library names are used for identification by the Xilinx tools.  Thus,

when compiling the schematic library source files, the target library

must have the same filename as the source file with the .SRC extension

replaced by .LIB.  For example, when compiling the xc4000 library, the

following command should be used to create xc4000.lib:

composer xc4000.src xc4000.lib

NOTE: versions of OrCAD prior to 386+ (ie, OrCAD 4.20) are limited in

memory usage.  As such, the enclosed library source files might fail

during compilation.  The suggested workaround is to split the source

files into smaller libraries.  When splitting the libraries, be sure

to retain the original library name on one of the sub-libraries.  For

example, to partition xc3000 into smaller libraries:

split source file using text editor:

xc3000.src -> xc3000x.src, xc3000y.src xc3000z.src

compose source files into sub-libraries:

composer xc3000x.src xc3000.lib

composer xc3000y.src xc3000y.lib

composer xc3000z.src xc3000z.lib

DISCLAIMER: these source files are provided to customers so they may

continue using OrCAD 4.20 until upgrading to OrCAD 386+.  This is

intended to be a temporary workaround.  Customers should upgrade to

OrCAD 386+ as soon as possible so that the libraries shipped with the

DS35 OrCAD Interface package can be used.  The use of sub-libraries

partitioned from the source files has not been thoroughly tested in

the Xilinx design flow.

Regards,

Xilinx Technical Support