First draft: 11/18/94

=====

Enclosed are the library source files for the Xilinx-OrCAD interface.

These source files are for the Xilinx XACT 5.0 Unified Libraries.  The

files enclosed are:

2000src\

xc2000.dsf      xc2000 simulation model source file

3000src\

xc3000.dsf      xc3000 simulation model source file

4000src\

xc4000.dsf      xc4000 simulation model source file

xmem2.dsf       memory modules simulation source file

7000src\

xc7000.dsf      xc7000 simulation model source file

The simulation model files shipped with XACT 5.0 are compatible

with OrCAD VST v1.10.  They are NOT compatible with VST v1.20.

For simulation with VST v1.20, the above device source files (.DSF)

must be compiled using the VST v1.20 tools.  The following instructions

describe the steps to compile the source files for a board level

simulation (similar steps are applicable for chip level simulation):

1) Copy the empty versions of the model libraries provided by OrCAD to

the board design directory.

COPY \ORCADESP\VST\LIBRARY\PRIMITIV.DAT \ORCAD\BOARD

COPY \ORCADESP\VST\LIBRARY\PRIMITIV.NDX \ORCAD\BOARD

2) Start the OrCAD tools and go to the Digital Simulation Tools menus.

Use CONFIGURE DIGIAL SIMULATION to adjust the Library Prefix to the board

design subdirectory \ORCAD\BOARD.

3) Execute CLEAR MODEL LIBRARY to prepare the new "empty" model libraries.

4) Use ADD DEVICE MODEL to prepare the Xilinx simulation models.  Use the

Prefix/Wildcard under the Local Configuration of ADD DEVICE MODEL to specify

the appropriate Xilinx device source file (.DSF).  If performing mixed

family simulations, repeat this step for each family DSF model file.

5) Use ADD DEVICE MODEL to prepare any other models necessary for board

related simulation models.

Note: Xilinx simulation models use 0.1 ns units.  Other board related

models may use different units.  Use the Specify Numerator/Denominator

For Scaled Delays processing option to synchronize the time parameters.

NOTE: the above instructions were taken from the beta copy of the OrCAD

VST v1.20 tutorial.  For the most up to date procedures or additional

information about VST, refer to the OrCAD documentation.

Regards,

Xilinx Technical Support