# Introduction to ViewSynthesis VHDL

Overview

What is VHDL?

ViewSynthesis for Windows synthesizes using the hardware description language VHDL (Very High Speed Integrated Circuit Hardware Description Language), and uses design files with a .VHD extension.

VHDL is the standard IEEE digital hardware design language that is the most widely used design language in both the commercial and military sectors of the electronics industry. You can obtain the IEEE Standard VHDL Language Reference Manual (ANSI/IEEE Std 1076-1993). The IEEE reference contains the definition of the VHDL standard, of which this is a subset. The manual provides a full BNF (Backus-Naur Form) language presentation and is the final authority on correctness. It is available from the IEEE Service Center, 445 Hoes Lane, P.O. Box 1331, Piscataway, NJ 08855-1331 U.S.A.

VHDL’s formal notation can express the function and hierarchical organization of a digital system such as Field Programmable Gate Arrays (FPGAs). A VHDL hardware design description includes two parts:

- Entity declaration that defines the interface (input and output ports of the circuit)

- Architectural body that defines the circuit’s operation

What is Logic Synthesis?

Logic synthesis is the process of turning a language description into a network of gates optimized for timing and area. The language description is a set of statements that represent state machines, data flow, control flow, logic equations or structure. It is written in a hardware description language, such as VHDL. Logic synthesis is followed by FPGA or ASIC place and route, just as if the gates created by synthesis had been entered by schematic capture.

The main benefit to using logic synthesis is reduced design entry time. Although expert gate level designers can occasionally manually design a circuit which is slightly smaller or faster than a synthesized circuit, synthesis is orders of magnitude faster. Each design change can be produced in minutes instead of days or weeks. Additionally, capturing a design using an HDL provides better documentation, and since it is technology–independent, it is easy to migrate the design to a different technology.

The remainder of this chapter provides an introduction into the basic VHDL syntax that ViewSynthesis for Windows requires.

About the VHDL Source File

A VHDL source file consists of a series of statements, using the following rules.

- Each statement is a series of words consisting of letters, digits, and punctuation characters.

- Words are terminated and separated by any number of blanks, tabs, and new lines.

- Statements are terminated and separated by semicolons (;).

- In some statements, lists of items may appear; items on such lists are separated by commas (,) or semicolons (;).

Comments may be included in Viewlogic VHDL source files by preceding them with two dashes (––). Whenever two consecutive dashes appear in a VHDL source, the remainder of the current line is a comment.

Base Specifiers

### Definitions

| Basic Specifier | Definition |
| --- | --- |
| B (Binary) | Extended digits in the bit value are 0 and 1. |
| O (Octal) | Extended digits are legal digits of octal numbering system, i.e., the digits 0 through 7. |
| X (Hex) | Extended digits are all digits, together with letter A through F. |

### Examples

signal DATA : BIT_VECTOR (11 downto 0) := 
	B”000011110101”;	–– or
	O”0365”;   			–– equivalent to B”000_011_110_101” or
	X”0F5”; 				–– equivalent to B”0000_1111_0101”

Note:

Throughout this reference, VHDL reserved words are displayed in lower-case, bold Courier type. See next section for list of reserved words.

VHDL Reserved Words

You cannot use a VHDL reserved word as an explicitly declared identifier. The following is a list of VHDL reserved words.

| abs | else | label | package | then |
| --- | --- | --- | --- | --- |
| access | elsif | library | port | to |
| after | end | linkage | postponed | transport |
| alias | entity | literal | procedure | type |
| all | exit | loop | process | unaffected |
| and | file | map | pure | units |
| architecture | for | mod | range | until |
| array | function | nand | record | use |
| assert | generate | new | register | variable |
| attribute | generic | next | rem | wait |
| begin | group | nor | report | when |
| block | guarded | not | return | while |
| body | if | null | rol | with |
| buffer | impure | of | ror | xnor |
| bus | in | on | select | xor |
| case | inertial | open | severity |  |
| component | inout | or | signal |  |
| configuration | is | others | shared |  |
| constant |  | out | sla |  |
| disconnect |  |  | sll |  |
| downto |  |  | sra |  |
|  |  |  | srl |  |
|  |  |  | subtype |  |