# Data Types

Predefined Types

Data Types Supported by Synthesis

ViewSynthesis for Windows supports VHDL data types as shown in Table 3-1. This chapter describes how to use each supported type in the following major sections:

- Predefined Data Types

- Type BOOLEAN

- Type INTEGER

- Type STD_LOGIC

- Type STD_ULOGIC

- Type TIME

- User-Defined Types

- Other Types

- Type Conversion

Table 3-1. Synthesis -supported Data Types

| Data Types | Fully Supported* | Conditionally Supported | Not Supported | Conditionally Ignored |
| --- | --- | --- | --- | --- |
| BOOLEAN | z |  |  |  |
| BOOLEAN_1D | z |  |  |  |
| STRING |  | z |  |  |
| INTEGER |  | z |  |  |
| INTEGER_1D |  | z |  |  |
| STD_LOGIC | z |  |  |  |
| STD_LOGIC_VECTOR | z |  |  |  |
| STD_ULOGIC | z |  |  |  |
| STD_ULOGIC_VECTOR | z |  |  |  |
| STD_LOGIC_2D |  | z |  |  |
| STD_ULOGIC_2D |  | z |  |  |
| BIT | z |  |  |  |
| BIT_VECTOR | z |  |  |  |
| CHARACTER |  | z |  |  |
| CHARACTER_1D |  | z |  |  |
| TIME |  |  |  | z |
| ENUMERATION | z |  |  |  |
| RECORD |  |  | z |  |
| RECORD-ARRAY |  |  | z |  |
| TEXT |  |  | z |  |
| User-defined arrays of scalar types |  | z |  |  |

* Fully Supported: Same usage as VHDL for simulation.

Predefined Data Types

Viewlogic’s synthesis supports the following VHDL predefined data types either fully, or conditionally, as shown in Table 3-1:

- BOOLEAN. This type represents one logical bit. BOOLEANs contain FALSE or TRUE.

- INTEGER. This type can be a maximum of 32 bits. INTEGERs contain values in the range -2147483647 to 2147483647.

- CHARACTER. This is an enumeration type (a type defined by the enumeration of its values), whose values are the 128 characters of the ASCII character set.

- BIT. This represents one logical bit (0 or 1).

- BIT_VECTOR. This represents an array of type BIT.

IEEE 1164 Defined Data Types

Viewlogic’s synthesis supports the following VHDL data types, as defined in IEEE Package 1164.

- STD_LOGIC. This type represents one resolved data bit. STD_LOGIC and can contain X, Z, 0, 1, L, H, W, U, or -.

- STD_LOGIC_VECTOR. This type represents an array of type STD_LOGIC.

- STD_ULOGIC. This type represents one unresolved data bit. STD_ULOGIC and can contain X, Z, 0, 1, L, H, W, U, or -.

- STD_ULOGIC_VECTOR. This type represents an array of type STD_ULOGIC.

User-Defined Types

- Emulation types

- Array of supported scalar types

In VHDL for synthesis, every VHDL object and value is of one of these base types or a user-defined enumeration type, or is an array composed of base types and/or user-defined enumeration types.

IMPORTANT:

1. Because STD_LOGIC and STD_ULOGIC usage are similar (except in the case of tristate modeling), the term “STD_LOGIC” or “STD_LOGIC_VECTOR” is used to represent both STD_LOGIC and STD_ULOGIC when usage is discussed in this manual.

2. ‘U’, ‘X’, and ‘W’ are used to define the behavior of the model itself, rather than the behavior of the hardware. These values should be avoided in synthesis models. ViewSynthesis for Windows interprets ‘X’ and ‘W’ as “don‘t cares,” and ‘U’ as zero (0).

3.  ‘L’ is interpreted the same as ‘0’.
     ‘H‘ is interpreted the same as ‘1’.
     ‘-’ is interpreted the same as ‘X’.

Arrays

Arrays are either one-dimensional (addressed with a single integer index) or two-dimensional (addressed with two integer indices). Two-dimensional arrays are partially supported.

Index ranges may be either ascending or descending. Ascending ranges proceed from n to m, while descending ranges proceed from m downto n, where m _ n. Index ranges may cover any range of integers, greater than or equal to zero.

Several array types are predefined and available in VHDL for synthesis. These may be used to declare objects that are arrays of the base types. The supported predefined array types are:

STD_LOGIC_VECTOR
STD_ULOGIC_VECTOR
STRING
BOOLEAN_1D
BIT_VECTOR
CHARACTER_1D
STD_LOGIC_2D

Type STD_LOGIC_VECTOR is a one-dimensional array of bits, indexed by integer values greater than or equal to zero.

Type STRING is an unconstrained one-dimensional array of CHARACTER. In VHDL for synthesis, STRING is restricted to use only in an ASSERT statement, and attribute values.

BOOLEAN_1D is a one-dimensional array of BOOLEAN.

STD_LOGIC_2D is a two-dimensional array of type STD_LOGIC_VECTOR. In VHDL for synthesis, its use is restricted to defining the dimension of constants used in a pla_table. (See Appendix B.)

The direction and bounds of index ranges are not constrained by the definitions of these types, and must be specified when objects of these types are declared.

Records are not currently supported for synthesis.

Type Boolean

About BOOLEAN Types

BOOLEAN is an enumeration type whose values are FALSE and TRUE.

Constants, variables, and signals may be of type BOOLEAN. Such objects may only assume values of type BOOLEAN.

### Boolean Example

process (A, B)
  variable COND1 : BOOLEAN;
begin
  COND1 := A > B;
  if COND1 then
    OUTPUT <= ’1’;
  else
    OUTPUT <= ’0’;
  end if;

end process;

Relational Operators

Values of type BOOLEAN may appear in expressions. The relational operators ( =, /=, <, <=, >, >=) are defined for operands of type BOOLEAN and one-dimensional arrays of BOOLEAN elements; they produce BOOLEAN results. FALSE is less than TRUE.

(As for all enumeration types, relational operators operate on one-dimensional arrays of type BOOLEAN by comparing their left-most elements; if these are equal, the operators will operate on their remainders.)

Logical Operators

The logical operators (and, or, nand, nor, xor, and not) are defined for operands of type BOOLEAN and one-dimensional arrays of BOOLEAN elements; left and right operand types and sizes must match. The result of each of these operators is of the same type and size as its operands.

Concatenation Operator

The concatenation operator (&) is also defined for operands of type BOOLEAN and one-dimensional arrays of type BOOLEAN. Its result is a one-dimensional array whose elements are of the same type as its operands’ elements.

Other Operators

Other expression operators are not defined for operands of type BOOLEAN.

Type INTEGER

About INTEGER Types

Values of type INTEGER represent 32-bit integers in the range -2147483647 to 2147483647. Constants, variables, signals, generic constants, and constant and variable subprogram parameters may be of type INTEGER. Such objects may only assume values of type INTEGER.

Literals of type INTEGER take the form:

[ base # ] digit { [ _ ] digit} [ # ]

The default base of INTEGER literals is 10; other legal bases are 2, 8, and 16.

Single underscores may appear within INTEGER literals; they do not affect resulting values.

Example

constant MINIMUM:     INTEGER := 0;

constant MAXIMUM:     INTEGER := 16#FF#;

INTEGER Ranges

An integer type consists of a set of integer values on which there is a range constraint. The range constraint takes the form:

range constant_simple_expression to constant_simple_expression

Each constant_simple_expression must be an integer-valued literal.

For minimum area, you should explicitly declare sizes (index_ranges) for all ports, signals and variables. If you omit a range specification, 32 bits is assumed. Integer sub-ranges must have literal bounds:

signal X:INTEGER range 0 to 7;

The most efficient implementation is when the range is specified as:

range 0 to 2n-1  or  range -2n to 2n-1

Operators

### Relational Operators

Values of type INTEGER may appear in expressions. The relational operators
 ( =, /= , <, <= , > , >= ) are defined for operands of type INTEGER and produce BOOLEAN results.

### Arithmetic Operators

The arithmetic operators (+, -, abs) are allowed on operands of type INTEGER, producing INTEGER results.

Multiplication, division, mod and rem (*, /, mod, rem) for synthesis follows these rules:

- Multiplication, division, mod or rem of two operands that are constants is allowed.

- Multiplication, division, mod, or rem is supported if the second operand is a constant, and is a power of 2. (example 2n, where n = 0,1,2,3...)

- Multiplication, division, mod or rem is not supported if both operands are signals or variables.

Exponential calculations (**) are not supported.

### Other Operators

Other expression operators are not supported for operands of type INTEGER.

Type Conversion

Type conversion between INTEGERs and one-dimensional STD_LOGIC_VECTOR and BIT_VECTOR arrays is defined. Synthesis supported conversion functions are listed in Appendix B.

Type STD_LOGIC (STD_ULOGIC)

IMPORTANT:

Because STD_LOGIC and STD_ULOGIC usage are similar (except in the case of tristate modeling), the term, “STD_LOGIC” or “STD_LOGIC_VECTOR” is used to represent both STD_LOGIC and STD_ULOGIC when usage is discussed in this manual.

About STD_LOGIC Types

Signals, constants, and variables may be of type STD_LOGIC.

STD_LOGIC is an enumeration type whose values are ‘X’, ‘Z’, ‘0’, ‘1’, ‘L’, ‘H’, ‘W’, ‘U’, or ‘-’. See page 3-3 for definitions of these values.

NOTE:

Avoid using ‘W’ or ‘U’ in synthesis models. They are essentially meant for simulation.

‘U’, ‘X’, and ‘W’ are used to define the behavior of the model itself, rather than the behavior of the hardware. These values should be avoided in synthesis models. ViewSynthesis for Windows interprets ‘X’ and ‘W’ as “don‘t cares,” and ‘U’ as zero (0).

For synthesis, STD_LOGIC value ‘L’ is equal to ‘0’ and ‘H’ is treated the same as ‘1’.

Special Treatment of ‘W’, ‘X’ and ‘-’ in Synthesis

ViewSynthesis for Windows treats ‘W’, ‘X’ and ‘-’ as “don’t care,” as shown in the next example. Because the meaning of ‘W’, ‘X’ and ‘-’ are different in simulation, differences may occur between VHDL simulation and post-synthesis netlist simulation.

### Example Using ‘X’ as Don’t Care

if ( S(0 to 5) = ”1X1X00” )

Is equivalent to:

if ( S(0) and S(2) and not S(4) and not S(5))

Where the X’d bits S1 and S3 are ignored.

Special Treatment of ‘Z’ in Synthesis

‘Z’ means “high impedance” in both simulation and synthesis. However, synthesis cannot compare to ‘Z’ as shown in the next example.

Conditional Support of ‘Z’ in Synthesis

- ‘Z’ may be used in concurrent conditional signal assignments to specify the tri-state conditions for target signals.

- ‘Z’ is not supported in sequential statements.

- ‘Z’ is not supported in operations, except in concatenation operations as shown in the following example.

S(0 to 3) <= ”11” & ”ZZ” when C1
else (”ZZ” & ”00”);

Relational Operators

Values of type STD_LOGIC may appear in expressions. The relational operators ( =, . <, <=, >, >= ) are defined for operands of type STD_LOGIC and one-dimensional arrays of STD_LOGIC elements (STD_LOGIC_VECTOR); they produce STD_LOGIC or STD_LOGIC_VECTOR results as appropriate. Both operands must have the same length.

Unsigned operands are assumed for relational operators.

See Chapter 5 for more information on relational operations on STD_LOGIC operands.

(As for all enumeration types, relational operators operate on one-dimensional arrays of type STD_LOGIC by comparing their left-most elements; if these are equal, the operators will operate on their remainders.)

Logical Operators

The logical operators (and, or, nand, nor, xor, and not) are defined for operands of type STD_LOGIC and one-dimensional arrays of STD_LOGIC elements (i.e. STD_LOGIC_VECTOR); left and right operand types and sizes must match. The result of each of these operators is of the same type and size as its operands.

Concatenation Operator

The concatenation operator (&) is also defined for operands of type STD_LOGIC and one-dimensional arrays of type STD_LOGIC. Its result is a one-dimensional array whose elements are of the same type as its operands’ elements.

Arithmetic Operators

The arithmetic operators (+, -) are defined for STD_LOGIC_VECTOR. If the operators are of different lengths, the shorter operand will be filled with zeros to match the length of the longer one.

Other Operators

Other expression operators are not defined for operands of type STD_LOGIC.

Type TIME

About TIME Types

In synthesis, TIME is either ignored or produces an error. The use where TIME is ignored is shown in the example below. All other uses of TIME in synthesis produce errors.

In the example, the CONSTANT current_delay declaration, TIME information and the AFTER expression are ignored. However, the statement “output <= input” is used for synthesis.

### Time Example

constant CURRENT_DELAY: TIME := 5 ns;
. . .
OUTPUT <= INPUT after CURRENT_DELAY;

User-Defined Types

About User-Defined Types

Type declarations are allowed in package declarations, package bodies, subprogram, architecture, block, and process declarative parts.

Types declared in packages may be imported for use by other entities via USE clauses.

After a type has been declared, its name may be used wherever predefined type names may be used in object declarations, and in type qualification operations.

Valid User-Defined Types

The following classes of user-defined types are allowed:

- Enumeration

- Array

These types will be discussed in detail on the following pages.

Enumeration Types

About Enumeration Types

Enumeration types are defined by the enumeration of their literals.

Objects declared to be of enumeration types may take on only the values defined by the enumeration symbols of their types. Such objects may be referenced by their simple names; e.g., “OP_CODE”.

Enumeration types are useful in the design of state machines.

Constraints

Support of enumeration arrays is restricted to one-dimensional arrays. An array of an array is not supported.

### Enumeration Type Format

Enumeration type declarations appear as follows:

type type_name is (enumeration_symbol{ , enumeration_symbol} ) ;

### Enumeration Type Example

type OP_TYPE is (OPADD, OPOR, OPAND, OPXOR) ;
variable OP_CODE: OP_TYPE;
.
OP_CODE := OPOR;

Relational Operators

Values of user-defined enumeration types may appear in expressions. The relational operators ( =, /=, <, <=, >, >= ) are defined for user-defined enumeration types. The order relations between enumeration values are defined by the order of their appearance in their type declaration--each value is equal to itself and less than its successor in the list.

### User-Defined Enumeration Types

Type conversion to and from user-defined enumeration types is not defined.

Concatenation Operator

The concatenation operator (&) is also defined for operands of enumeration types and one-dimensional arrays of enumeration types; its result is a one-dimensional array whose elements are of the same type as its operands’ elements.

State Encoding

ViewSynthesis for Windows has a setting for ONE HOT encoding of state machines. This setting causes enumeration types used in VHDL to be encoded ONE HOT rather than SEQUENTIAL. For example:

type STATE_TYPE is (START, SA, SB, SC, SD, SE, SF, SG);
signal CURRENT_STATE :STATE_TYPE;

In ViewSynthesis for Windows, if a design containing this line is compiled with the encoding set to ONE HOT, CURRENT_STATE will be an eight-bit vector. If the encoding type is set to SEQUENTIAL, CURRENT_STATE will be a three-bit vector.

When a ONE HOT encoded signal or variable is used in a case statement, = expression, or 
/= expression, only the ONE HOT bit is tested.

If you want to code a state machine using ONE HOT encoding, you must use an enumeration type for the state variable.

Array Types

About Array Types

Array types are composite types. An array consists of a group of elements of identical type which are indexed by position. Array types are defined by the type of elements they contain and the number 
and/or range of their indices. Array type declarations declare either constrained or unconstrained array types.

- A constrained type declaration defines the dimension and size of every object of its type.

- An unconstrained type declaration defines the dimensionality of objects of its type without defining their sizes.

Each range (for a constrained array type) or index type definition (for an unconstrained array type) specifies one dimension of the declared array type. (Only INTEGER ranges are supported.)

### Array Type Format

Constrained array type declarations appear as follows:

type type_name is
  array (index_range) 
  of scalar_type;

Unconstrained array type declarations appear as follows:

type type_name is
	array  (INTEGER range <> )
		of scalar_type;

NOTE:

index_range ::= m to n or n downto m.
where m and n are integer constants and m _ n.

scalar_type ::= BOOLEAN or STD_LOGIC

Array Type Example

type OP_TABLE_TYPE      is array (1 to 4) of OP_TYPE;
type DATA_MEMORY_TYPE   is array (INTEGER range <> of OP_TYPE;
variable OP_TABLE:      OP_TABLE_TYPE;
variable DATA_MEMORY:   DATA_MEMORY_TYPE (0 to 255);
.
.
.
DATA_MEMORY(5) := OP_TABLE(3);

Array Elements

An object that is a one-dimensional array has one element for each possible index value.

All elements of an array are of the same type and size. Array objects can be referenced by their simple names (“current_word”), indexed names (“memory(225)”), or slice names 
(“current_word(3 to 5)”).

In general, arrays of arrays and multi-dimensional arrays are not supported in synthesis with one exception:

STD_LOGIC_2D is supported only when used with the pla_table function. (See Appendix B.)

Logical Operators

Values whose types are user-defined array types may appear in expressions. The logical operators (and, or, nand, nor, xor, and not) are defined for one-dimensional arrays of STD_LOGIC, BIT, or BOOLEAN elements. The result is an array of the same type and size. Both operands must be the same length.

Ordering and Equality Operators

The ordering and equality operators (<, <=, >, >=, =, /=) are only defined for one dimensional arrays of the same length. This does not apply to INTEGERs and INTEGER ranges.

Concatenation Operator

The concatenation operator (&) is also defined for operands of one-dimensional array types. Its result is a one-dimensional array whose elements are of the same type as its operands’ elements.

Other Operators

Other expression operators are not defined for user-defined array types.

Severity Level Types

Severity level types can be used in ASSERT statements. These statements are used for simulation, and are ignored by synthesis. Severity Level types include NOTE, WARNING, ERROR, and FAILURE.

Type Conversion

Matching Types

Data types of operands in expressions, assignments, and other statements must exactly match expected operand types. VHDL includes type conversion operators for explicit conversion of operand types.

Cautions When Using STD_LOGIC Values

It is important to note that values of type STD_LOGIC can affect the results of expression evaluation differently from the conventional logic values 0 and 1. When an STD_LOGIC object containing 0 or 1 is converted to a BOOLEAN result, the result is FALSE or TRUE, as expected.

When a BOOLEAN value is converted to an STD_LOGIC, the result is 0 for FALSE and 1 for TRUE.

When a BOOLEAN value is converted to an INTEGER, the result is 0 for FALSE and 1 for TRUE.

Zero Extension and Truncation

Zero extension and truncation are used when converting two objects of different sizes.

### Zero Extension Example

NUM := V1D2INT(V1);  -- where NUM is 32-bits, and V1 is 2-bits

### Truncation Example

variable NUM : INTEGER;
variable V1: STD_LOGIC_VECTOR (1 downto 0);
.
.
.
V1 := INT2V1D(NUM) --where V1 is 2-bits, and NUM is 32-bits