# Sequential Statements

About Sequential Statements

Sequential statements define the algorithms that express the behavior of design entities.

Sequential statements appear in process statements and subprograms. They are executed sequentially, in their order of appearance.

WAIT Statement

What the “wait” Statement Does

The wait statement suspends a process or procedure.

When a wait statement is executed, the process that contains it (or which is the parent of the procedure that contains it) is suspended. Execution resumes when any signal in the condition changes value, and the condition is evaluated to be TRUE.

A wait statement may not appear within functions, and may not be executed by procedures called directly or indirectly by functions.

### wait Statement Format

wait until condition;

Using the WAIT Statement

The wait statement must be the first statement in a clocked process. It is used to specify the clocked sequential circuit as shown in the example below. When the clock edge, or asynchronous reset or set conditions are TRUE, the process continues.

### wait Statement Example

process
wait until (CLK’EVENT and CLK = ’1’) or (RESET = ’1’));
  if (RESET = ’1’) then
    BCD <= ”0000”;
  else
    BCD <= MUXZ;
  end if;
end process;

Assertion Statement

Assertion Statement Ignored

The assertion statement can be included in the VHDL description to check that a specified condition is true. However, it is ignored in the synthesis process.

Signal Assignment Statement

What the Signal Assignment Statement Does

The signal assignment statement assigns an expression to a signal.

Reassignment to the same signal is allowed, and the last signal assignment defines the signal’s final value in a process.

The transport delay is not supported in synthesis.

### Signal Assignment Statement Example

SS1 <= A and B;
	if COND1 then
		SS1 <= C and D;
	end if;

Variable Assignment Statement

What the Variable Assignment Statement Does

The variable assignment statement assigns the value of an expression to a variable. The old value of the variable is replaced with the new value, and the value is updated immediately.

### Variable Assignment Statement Format

variable_name := expression;

### Variable Assignment Statement Example

COUNTER := COUNTER + 1;

Variables have a single or composite value, which changes whenever an assignment is made to the variable. Variables may be of any supported type. Variables assigned and used in the same state of a clocked process will be just a wire. Variables assigned in one state and used in another will be registers. Variables in combinational processes must be initialized before use and will represent a wire.

Procedure Call Statement

What the Procedure Call Statement Does

Procedure call statements invoke procedures. The procedure call statement consists of the name of the sub-program to be invoked, and a list of its parameters

An actual parameter is an expression, variable name, or signal name. Parameter association is made by position.

See the discussion of subprograms in Chapter 2 for more information on procedures.

### Procedure Call Statement Format

procedure_name (actual_parameter {, actual_parameter});

### Procedure Call Statement Example

INITIALIZE (DATA, LENGTH, X”0000”);

IF Statement

What the “if” Statement Does

An if statement allows conditional execution of sequential statements. It includes:

- A Boolean expression that is the condition to test

- A sequence of statements to execute if the condition is TRUE

- Zero or more elsif conditions and statement sequences

- An optional final else sequence

The Boolean conditions are evaluated until one evaluates to TRUE or the end of the statement is reached. The else is equivalent to “elsif FALSE”. The first sequence of statements associated with a TRUE condition is executed; if none of the conditions is TRUE, no statements are executed.

### if Statement Format

if condition then
	sequence_of_statements

{elsif condition then
	sequence_of_statements}
[else
	sequence_of_statements] 
end if;

### if Statement Example

if ENABLE = ’0’ then
	OUTPUT <= ’0’;
elsif READING then
	OUTPUT <= INPUT;
else
	OUTPUT <= ’X’;
end if;

If the conditions are not completely specified (as shown below) latches are generated to hold the value of the target signal.

if C1 = ’1’ then
	TARGET_SIGNAL <= A or B;
end if;

Latch Implementation

About Storage Elements in “if” and “case” Statements

ViewSynthesis for Windows will analyze the source code, and attempt to determine where latched (in unclocked processes) and register recirculation (in clocked processes) are needed. An if condition is considered to be completely specified when the last branch is specified with an else clause. A signal is considered to be exhaustively assigned if a signal is:

- unconditionally assigned

- assigned in every branch of an “if ... elsif ... else” statement

- assigned in every branch of a case statement and that case statement is complete. While VHDL specifies that case statements must be complete ViewSynthesis for Windows does not enforce this.

When a signal is exhaustively assigned in a process, no latch or register recirculation is needed or implemented.

Users are encouraged to completely specify signal assignments. This is particularly important when using one hot state encoding.

The example:

process(A, B)
begin
	if (CLOCK = ’1’ and CLOCK’EVENT) then
		case STATE is
			S1 | S3 => SIG <= ’1’;
			S2 => if FOO = ’1’ then
					SIG <= ’0’;
				end if;
		end case;
	end if;
end process;

will produce a recirculation condition of (not (S1 or S3 or (S2 and FOO))). If encoding is complete (sequential encoding with 2n states) this is an optimal result. If not, the recirculation condition includes illegal states. It would be better coded as:

process(A, B)
begin
	if (CLOCK = ’1’ and CLOCK’EVENT) then
		case STATE is
			S1 | S3 => SIG <= ’1’;
			S2 => if FOO = ’1’ then
					SIG <= ’0’;
				else
					SIG <= SIG;
				end if;
		end case;
	end if;
end process;

This will produce a recirculation condition of S2 and not FOO.

Treatment of ‘X’, ‘W’ and ‘-’

The STD_LOGIC values of ‘X’, ‘W’ and ‘-’ are treated as “don’t care” as shown in the below example:

if ( A ( 3 downto 0) = ”11X0”)

Is the same as:

if ( A(3) and A(2) and not A(0))

NOTE:

‘Z’ is not supported in sequential statements.

CASE Statement

What the “case” Statement Does

A case statement is similar to an if statement and also provides conditional execution of sequential statements. The case statement consists of an expression to evaluate, and a list of statement sequences. Each sequence is associated with a list of choices whose values determine whether the sequence is executed.

Determining Matches

When the case statement is executed, the given expression is evaluated, and the result is compared to the choices until a match is found. When the given expression’s value is found in a list of choices, the sequence of statements following the list is executed. The statement sequence is preceded by a selection operator (=>). All case conditions must be mutually exclusive.

The vertical bar (|) indicates that any of the given choices qualifies as a match. Every choice must be an expression.

The selection expression must evaluate to an INTEGER array or enumeration type; it may not be a record. All members of the expression value’s type must be accounted for in a when clause, and no member may appear in more than one when clause. All choice values must be known at analysis.

The choice others is required when the explicit choices before it are not exhaustive, following the same rule as for simulation.

### case Statement Format

case expression is
	when choices => sequence_of_statements;
	{when choices => sequence_of_statements;}
	when others => sequence_of_statements;
	end case;

Any number of when clauses may be included. Choices take the form:

choice { | choice}

### case Statement Example

case OP_CODE is
	when ADD  			=> RESULT := OP1 + OP2; 
	when SUB  			=> RESULT := OP1 - OP2; 
	when ZERO | NOP 	=> RESULT := 0; 
	when others   		=> ERROR  := ’1’;
end case;

LOOP Statement

What the “loop” Statement Does

The loop statement is a sequence of statements to be executed some number of times.

The for loop executes once for each value in the range. During each iteration, this value is assigned to the variable declared by the loop statement. Its value may not be assigned within the loop. If the given range is null, e.g. “1 downto 10”, the loop is not executed.

A while loop is supported in standard VHDL and are not supported in synthesis.

### loop Statement Format

[loop_label:]
 for loop_index_variable in range loop
	sequence_of_statements
end loop [loop_label];

### loop Statement Example

variable A: STD_LOGIC_VECTOR (7 downto 0);
.
.
.
RESULT := ’0’;
for I in A’RANGE loop -- same as ”7 downto 0”
	RESULT := RESULT xor A(I);
end loop;

RETURN Statement

What the “return” Statement Does

The return statement returns a value when it appears within a function. In synthesis, return is only supported in a function body, and it must be the last statement in the function body to return a result.

### return Statement Format

return expression;

### return Statement Example

return sum;

NULL Statement

What the “null” Statement Does

A NULL statement is an empty statement; it performs no action.

### null Statement Format

null;