# An Introduction to VHDL and Synthesis

Overview

If you’re completely new to software that is used to design hardware, you are faced with a flood of new words and concepts you don’t completely understand yet. Hopefully, this tutorial will provide you with the information you need to start writing VHDL hardware models that you can synthesize into gates. It is basically a walk through of a handful of small, real world examples and the way in which you should look at them. Although this appendix may appear to start at a basic level, it gets more complex fairly quickly, so it pays not to skip any part of it.

In order to write VHDL that synthesizes well, as an engineer, you’ve got to learn the processes that a synthesizer goes through in coming up with an implemented design. A goal of this appendix is to give an understanding of basic VHDL and how to use it to make designs. After reading this, if you can successfully analyze VHDL you’ve never seen before and basically understand how it maps to gates, this appendix has achieved that goal.

VHDL Basic Rules

- VHDL is a Hardware Description Language (HDL) which can be used to accurately simulate designs. Synthesis is the process of translating correct, debugged VHDL designs into latches, combinational logic and flip-flops to make ASICs or FPGAs.

- VHDL is case insensitive. “FOOBAR” is interpreted exactly like “foobar”, “FooBar”, or “FoObAr”.

- VHDL ignores white space like blanks, tabs, newlines, etc. Feel free to format any way that pleases the eye.

- The use of VHDL reserved words other than intended is strictly verboten! No variables called “case” or signals called “select” are allowed

- VHDL is very unforgiving with “type” differences. (This means, for example, that you can’t mix “INTEGERs” and “STD_LOGIC_VECTORs.”)

- Most Viewlogic VHDL tools automatically access IEEE.STD_LOGIC_1164.all and SYNTH.PACK1164.all.

- Although there’s plenty of ways to write simulatable VHDL, many of these coding styles won’t translate to actual hardware via synthesis well. Subtle VHDL coding differences have a very large impact during synthesis.

Ways to Build Multiplexers

Although VHDL has a selection of gate level binary operators like “and”, “or”, “nand”, “nor”, “xor”, and “xnor”, a multiplexer (MUX) can come in an almost infinite variety of shapes and sizes. A random sample of MUXes could be: 5-bit 2:1, 8-bit 4:1, 2-bit 5:1, 37-bit 37:1, etc. Each of these requires a little more work to accurately describe in VHDL.

Figure A-1. The Block and Gate Level Model of a 1-bit 4:1 MUX

To create a behavioral VHDL listing for any flavor of multiplexers (or any other hardware design for that matter), you first have to include the standard VHDL libraries that let you model all the logic values you want to use. (The term used in VHDL for a library is “package.”) Although early VHDL packages offered only four logic basic values (’0’, ’1’, ’Z’ for high impedance, and ’X’ for unknown), current practice is to use the nine values:

’U’ -- Uninitialed,
’X’ -- strong Unknown,
’0’ -- strong 0,
’1’ -- strong 1,
’Z’ -- High Impedance,
’W’ -- Weak Unknown,
’L’ -- weak 0,
’H’ -- weak 1,
’-’ -- don’t care

Figure A-2. The Logic Values a VHDL signal or variable Can Assume as Supported by the IEEE.STD_LOGIC_1164 Package

These nine logic values are available in the IEEE.STD_LOGIC_1164 package. In the Viewlogic design environment there’s also a package useful for synthesis called SYNTH.PACK1164. To have all these packages available for your specific design you put at the beginning of your VHDL:

library		IEEE;
use			IEEE.STD_LOGIC_1164.all;
library		SYNTH;
use			SYNTH.PACK1164.all;

Figure A-3. How VHDL Packages (Libraries) Are Pulled into Designs

Having the IEEE.STD_LOGIC_1164 package now accessible, to consider anything as a nine logic value signal (or variable) you just reference STD_LOGIC as its type if it’s a single bit or STD_LOGIC_VECTOR(<bit range>) if it’s a bus. A good example of this comes up in the entity/port part of our MUXes.

entity MUX1 is
	port(D0, D1, D2, D3	: in  STD_LOGIC_VECTOR(3 downto 0);
			SEL					: in  STD_LOGIC_VECTOR(1 downto 0);
			ZOUT					: out STD_LOGIC_VECTOR(3 downto 0));
end MUX1;

Figure A-4. The Multiplexer’s entity and port Declarations

As far as hardware design is concerned, entity statements live just to hold port declarations. Ports can be one of four types: in -- for inputs, out -- for outputs, inout -- for bi-directional ports, and buffer -- for output ports that can be read internally. (That is, if an out port outputs a signal called “foo”, “foo” can’t be read or used internally. But if this port is changed to a buffer, “foo” can be output and used internally.  buffer outputs are useful for counters and state machines.)

One of the fundamental strengths (or weaknesses) of VHDL is that for any one entity declaration, an engineer can create one or more VHDL descriptions for that entity in a section of code called its architecture. An example of why this is powerful is that designers could create a gate level description of a design (which simulates slowly) and a separate purely behavioral description (which simulates quickly.) The reason why this is a weakness is that if a human created both architectures there’s no reason to believe they are functionally equivalent without extensive testing. (Hence one of the fundamental needs synthesis tools fill; designers can create and quickly debug fast running behavioral VHDL descriptions of their designs and then let the synthesis tools translate it to gates.)

In this following code segment we’re using the with select construct to build a 4:1 MUX. Notice that although this MUX is fully defined (i.e. every possible 2-bit combination of “SEL” is defined), there’s an others statement associated with “----”. Literally what this is telling the synthesis tool is that if you encounter an undefined “SEL” value, use “don’t cares” as the output. What this demonstrates is good default VHDL coding style. If, for example, you had a situation where you only needed a 3:1 MUX and didn’t care what happened at “ZOUT” when, say, SEL = “01”, this style (”----”  when others;) would enable gate level optimization using powerful “don’t cares”.

architecture BEHAV1 of MUX1 is
begin
  with SEL select ZOUT <=	D0  when ”00”,
                        	D1  when ”01”,
                        	D2  when ”10”,
                        	D3  when ”11”,
                        	”----”  when others;
end BEHAV1;

Figure A-5. Using VHDL’s Selected Signal Assignment to Build a 4:1 MUX

Below is the another way of implementing a 4:1 MUX using a case statement instead. Note the similar use of others and “----” (don’t cares) for all non-defined situations.

architecture BEHAV2 of MUX1 is
begin
  process (D0, D1, D2, D3, SEL)  -- note this sensitivity list
  begin
    case SEL is
      when ”00”    => ZOUT <= D0;
      when ”01”    => ZOUT <= D1;
      when ”10”    => ZOUT <= D2;
      when ”11”    => ZOUT <= D3;
      when others  => ZOUT <= ”----”;
    end case;
  end process;
end BEHAV2;

Figure A-6. Using a case Statement to Build a 4:1 MUX

In this code sample, a process statement is followed by all the input signals that are used by that process (called a “sensitivity list”.) That is, “ZOUT” is not in the sensitivity list because it’s an output by this process. Sensitivity lists are used to trigger when a process should be evaluated. In this example the sensitivity list specifically says: “when D0, D1, D2, D3, SEL change, re-evaluate the process statement.” If, “D2” was mistakenly left out of the sensitivity list, this VHDL would simulate and synthesize in a way different than in Figure A-6! As a rule, incomplete sensitivity lists can mean rather tricky simulate and synthesize results. (There’s more on this in the “Counting Down the Decades” section of this appendix.)

The fact that VHDL uses process loops a lot causes most designers to use selected signal assignments outside of process loops for small MUXes.  case statements inside process loops are used for large MUXes and/or MUXes involving complicated inputs.

It’s important to note that even though two different coding styles were used to model a 4:1 MUX, simulation and the gate level results of synthesis would be the same. A good coding habit is to choose one style and stick with it instead of randomly mixing both interchangeably. You’ll know what it does and not have to waste time rethinking styles to do the same task. One important synthesis rule is: “Even if I write in different coding styles, if I can get my VHDL to behave exactly in some manner, it will synthesize to the same function at the gate level.”

architecture BEHAV3 of MUX1 is
begin
  process (D0, D1, D2, D3, SEL)
    begin
      if (SEL = ”00”) then
        ZOUT <= D0;
      elsif (SEL = ”01”) then
        ZOUT <= D1;
      elsif (SEL = ”10”) then
        ZOUT <= D2;
      else
        ZOUT <= D3;
      end if;
    end process;
end BEHAV3;

Figure A-7. BAD CODE! Trying to Use an if-elsif Construct While Attempting to Build a 4:1 MUX

While this may appear to be the exact functionality of a 4:1 MUX, think again! There’s an implied order of evaluation in this if-elsif construct: first you must test “if (SEL = ”00”)” then if that fails, the simulator then tests “elsif (SEL = ”01”)”. What is actually described here, and built by the synthesis tool, is a priority encoder.

architecture BEHAV4 of MUX1 is
begin
  process (D0, D1, D2, D3, SEL)
  begin
    if (SEL(1) = ’0’) then
      if (SEL(0) = ’0’) then ZOUT <= D0;
      else ZOUT <= D1;
      end if;
    else
      if (SEL(0) = ’0’) then ZOUT <= D2;
      else ZOUT <= D3;
      end if;
    end if;
  end process;
end BEHAV4;

Figure A-8. Correct Usage of an if-then-else Construct to Build a 4:1 MUX

Careful examination yields that this does indeed model a 4:1 MUX. It’s SEL (select inputs) is broken down into SEL(1) and SEL(0). These select inputs are used individually in a nested if -then -else construct to mimic the 4:1 MUX behavior. Notice that although this style is less “readable” by humans (compared to Figure A-6’s case statement approach) it synthesizes to the same 4:1 MUX.

This teaches that you can describe digital circuits (in this case MUXes) in many different ways, but the gate level output of the synthesis tool will be functionally the same. The pitfalls come in accidentally writing VHDL that’s close to what you want but not exact. Pulling together the first complete VHDL design (for a 4:1 MUX) using Figures A-3, A-4, and A-6 would look like:

library		IEEE;
use			IEEE.STD_LOGIC_1164.all;
library		SYNTH;
use			SYNTH.PACK1164.all;

entity MUX1 is
  port (	D0, D1, D2, D3	:	in 	STD_LOGIC_VECTOR(3 downto 0);
        	SEL          	:	in 	STD_LOGIC_VECTOR(1 downto 0);
        	ZOUT         	:	out	STD_LOGIC_VECTOR(3 downto 0));
end MUX1;

architecture BEHAV2 of MUX1 is
begin
  process (D0, D1, D2, D3, SEL)  begin
    case SEL is
      when ”00”    => ZOUT <= D0;
      when ”01”    => ZOUT <= D1;
      when ”10”    => ZOUT <= D2;
      when ”11”    => ZOUT <= D3;
      when others  => ZOUT <= ”----”;
    end case;
  end process;
end BEHAV2;

Figure A-9. A Complete VHDL Design for a 4:1 MUX

Latches and Registers

The most basic memory element is the latch -- a device that holds a value until its told to change that value by a gating input called a latch enable. The VHDL that does this looks like an incomplete MUX.

process ( LATCH_ENABLE, DATA_IN )
begin
  if ( LATCH_ENABLE = ’1’ ) then
    QOUT <= DATA_IN;
  end if;
end process;

Figure A-10. The Basic VHDL That Synthesizes to a Latch

Notice how if “LATCH_ENABLE” is ’0’, “QOUT” doesn’t change -- it’s old value is retained. This code synthesizes to:

Figure A-11. The Synthesis Results of the Latching VHDL Code

One of the key points to get from this example is that “QOUT” is not given a value under all conditions, but only when LATCH_ENABLE = ’1’. This creates the need for the synthesis software to put down a latch in the gate level output to mimic this VHDL’s functionality exactly in hardware. (This is called latch inferencing in the VHDL.) Watch out, though! It’s possible to fool yourself into thinking you’re building resetable latches with VHDL like:

process ( LATCH_ENABLE, DATA_IN )
begin
  if ( LATCH_ENABLE = ’1’ ) then
    QOUT <= DATA_IN;
  else
    QOUT <= ’0’;
  end if;
end process;

Figure A-12. VHDL Code That Can Mislead You Into Thinking You’re Building a Resetable Latch

Which in reality, the synthesis software will see this functionality doesn’t involve latches, but a simple 2 input AND gate!

Figure A-13. But the Code Synthesizes to a 2 Input AND Gate!

There are two basic ways to describe registers in VHDL. Both techniques involve controlling what values a signal or a variable gets as a function of a clock. The simplest way is to use the wait until statement.

process
begin
  wait until (CLOCK’EVENT and CLOCK = ’1’);
    QOUT <= DATA_IN;
end process;

Figure A-14. The Simple Use of a wait until Statement to Build Registers

The “CLOCK’EVENT” part says “look for the clock to change”; the “and CLOCK = ’1’” part says “and look for the rising edge.” (Combined, these code fragments create what are called “edge expressions” in VHDL. That is, they become active on the edge of a signal or variable.) If you wanted negative edge registers you would change this to “and CLOCK =’0’”.

Figure A-15. Synthesis Results of wait Statements to Build a Register

One of the main drawbacks of this style of inferring registers is that the IEEE has a restriction that processes can’t have both sensitivity lists and wait statements. What this means is that you can’t use wait statements to build asynchronous or resetable registers. To infer registers you have to mirror the way latches were inferred (see Figure A-10) but use edge expressions in place of the latch enables.

process ( CLOCK, DATA_IN )
begin
  if ( CLOCK’EVENT and CLOCK = ’1’ ) then
    QOUT <= DATA_IN;
  end if;
end process;

Figure A-16. Using if and Edge Expressions to Infer Registers

This synthesizes to exactly the same gate level design shown in Figure A-15. If you remember the prior discussion of how process sensitivity lists are used to trigger a process’s evaluation, it’s also perfectly OK in VHDL to rewrite Figure A-16 to be:

process ( CLOCK )  -- ”DATA_IN” not included here!
begin
  if ( CLOCK’EVENT and CLOCK = ’1’ ) then
    QOUT <= DATA_IN;
  end if;
end process;

Figure A-17. “DATA_IN” Was Not Necessary in the process’s Sensitivity List!

This, too, synthesizes and simulates the same way Figures A-14 and A-16 do. Why? Because “DATA_IN’s” value isn’t important until “CLOCK’s” edge changes. That is, it makes no difference if “DATA_IN” changes 27 times between the rising edges of “CLOCK” in Figure A-17 so “DATA_IN” doesn’t need to be used to trigger the evaluation of the process loop.

To infer flip-flops with asynchronous sets or resets, parrot the following coding style using the if-elsif construct.

process ( RESET, CLOCK, DATA_IN )
begin
  if ( RESET = ’1’ ) then  -- asynchronous!
    QOUT <= ’0’;
  elsif ( CLOCK’EVENT and CLOCK = ’1’ ) then  -- synchronous!
    QOUT <= DATA_IN;
  end if;
end process;

Figure A-18. if-elsif to Infer Registers with Asynchronous Resets

When using this style it’s important to put all the asynchronous signals or variables before the final “elsif ( CLOCK’EVENT and CLOCK = ’1’ )”! The following VHDL may simulate equivalently (depending on how you test it):

process ( RESET, CLOCK, DATA_IN )
begin
  if ( CLOCK’EVENT and CLOCK = ’1’ ) then  -- synchronous!
    QOUT <= DATA_IN;
  elsif ( RESET = ’1’ ) then              -- asynchronous!
    QOUT <= ’0’;
  end if;
end process;

Figure A-19. Unbuildable if-elsif to Infer Registers with Asynchronous Resets

This is unbuildable because the priority encoder inherent in the if-elsif construct literally says: “if not on the rising edge of CLOCK, then reset QOUT.” (That is, it is impossible to build hardware that runs purely on the non-rising edge of CLOCK.) Figure A-18 (the correct coding style) synthesizes to:

Figure A-20. Synthesis Results of Figure A-18.

Like Figure A-17, Figure A-18’s “DATA_IN” can be removed from the process loop’s sensitivity list, but “RESET” must remain! Why? Because “RESET” is asynchronous; each time it changes it must trigger a new evaluation of the process loop.

process ( RESET, CLOCK )  -- no ”DATA_IN” but need ”RESET” here!
begin
  if ( RESET = ’1’ ) then  -- asynchronous!
    QOUT <= ’0’;
  elsif ( CLOCK’EVENT and CLOCK = ’1’ ) then  -- synchronous!
    QOUT <= DATA_IN;
  end if;
end process;

Figure A-21. All Clocks and Asynchronous Beasties Must Be on the process Loop’s Sensitivity List!

Very quickly these constructs can be used to create complicated hardware with both synchronous and asynchronous values driving things! In this example, “QOUT” and “DATA_IN” are 4 bits; “LOAD_5”, “RESET”, and “INCR_LOAD” are single bit and arrive asynchronously.

process ( LOAD_5, INCR_LOAD, RESET, CLOCK )
begin
  if ( RESET = ’1’ ) then  -- asynchronous!
    QOUT <= ”0000”;
  elsif ( LOAD_5 = ’1’ ) then  -- asynchronous, too!
    QOUT <= ”0101”;
  elsif ( CLOCK’EVENT and CLOCK = ’1’ ) then  -- synchronous!
    if ( INCR_LOAD = ’1’ ) then
      QOUT <= QOUT + ”0001”;
    else
      QOUT <= DATA_IN;
    end if;
  end if;
end process;

Figure A-22. An Untestable (Due to Unpredictable Timing) Yet Buildable Design that

Uses Synchronous and Asynchronous Sets and Resets

This builds a 4-bit register with “RESET” tied to each flip-flop’s RESET pin plus some logic tied to each register’s set and RESET pins to implement the asynchronous “LOAD_5” operation. On the D inputs of each flip-flop is a 2:1 MUX selecting (based on “INCR_LOAD”) either “DATA_IN” or “QOUT + 1”.

Figure A-23. BAD DESIGN STYLE! Synthesis Results of Figure A-22’s Scary, Yet Buildable Design

WARNING!

Although it is possible to have registers suddenly set or reset based on some input -- this design practice will lead to ASIC’s and FPGA’s that, at most times, are untestable without the extensive and painful use of expensive static timing analysis software.

Different Constructs Cause Different Results

A quick review of what you’ve just read should drive home how mostly minor differences using if-then-else, case, with-select, wait-until and if-elsif constructs can have a huge impact as to whether you’ll get multiplexors, priority encoders, latches, registers, or something unbuildable out of synthesis!

If you don’t conscientiously think about how the VHDL you code maps to gates, the synthesis tool won’t miraculously clean up your sloppy code. Just because you can get a design to simulate successfully doesn’t mean it can be synthesized to viable or efficient hardware.

Decade Counters

As anyone who lived through the 1960’s and 1970’s can tell you, digital readouts were cultural icons symbolizing a high tech future at that time. Science fiction movies filmed then went to great lengths to show digital readouts. Texas Instruments made a killing selling digital watches -- which are essentially battery powered Binary Coded Decimal (BCD) counters small enough to wear on a wrist. The state diagram for a BCD counter looks like:

Figure A-24. State Diagram Table for a BCD Counter

The interface we’ll use to define how our BCD counter interacts with the rest of the world looks like:

entity DECADE is
  port (	UP_ENABLE	: in    	STD_LOGIC;
        	CLK      	: in    	STD_LOGIC;
        	RESET    	: in    	STD_LOGIC;
        	QOUT     	: buffer	STD_LOGIC_VECTOR(3 downto 0);
       	 	COUNT_OUT	: out   	STD_LOGIC));
end DECADE;

Figure A-25. The entity and port Declarations for a BCD Counter

Note that “QOUT” is a buffer. This is because you want to be able to use it as an output yet be able to read its value inside of the design’s architecture for use in counting. It’s important to also know that everything between an architecture’s begin and end are considered parallel operations by synthesis and simulation. To get sequential behavior you must put your VHDL into a process loop.

architecture BEHAV of DECADE is 
begin
  process begin
    wait until (CLK’EVENT and CLK = ’1’);
    if (RESET = ’1’) then
      QOUT <= ”0000”;
    else
      if (UP_ENABLE)
        if (QOUT = ”1001”) then
          QOUT <= ”0000”;
        else
          QOUT <= QOUT + ”0001”;
        end if;
      else
        QOUT <= QOUT;
      end if;
    end if;
  end process;

  COUNT_OUT <=	’0’ when (UP_ENABLE = ’0’) else
             	’1’ when (QOUT = ”1001”) else ’0’;

end BEHAV;

Figure A-26. The Behavioral Body of the BCD Counter

Here you have two concurrent events: “COUNT_OUT <=” outside of the process loop, and the process loop itself. process loops fire up to evaluate what their outputs will become when one of the signals on their sensitivity lists changes. But in this case, there’s no sensitivity list! This is where the “wait until (CLK’EVENT and CLK = ’1’);” comes into play. What this does is says “wait until there’s a rising edge on ‘CLK’ and evaluate all the different ways ‘QOUT’ can be assigned a new value.”

Now comes the time when you start thinking like a synthesis tool. “QOUT” is registered since it’s the only signal being assigned a value in this process. “COUNT_OUT” appears to be the output of a 3:1 MUX with the when-else construct -- but closer examination yields some prioritization going on here so it’s a small 3 level priority encoder. Also, the “(UP_ENABLE = ’0’)” means “UP_ENABLE’s” inverted form is used and the “(QOUT = ”1001”)” implies a small comparator comparing “QOUT” to ”1001”. Check out from Figure A-26:

if (QOUT = ”1001”) then
                     QOUT <= ”0000”;
                   else
                     QOUT <= QOUT + ”0001”;
                   end if;

Figure A-27. Code that Builds a 4-bit 2:1 MUX with Comparator between “QOUT” and “1001”

Plus a 4-bit Incrementor on One Leg of the MUX

After some careful consideration you’ll start to see that what’s described in Figure A-26 is a 4-bit BCD up counter with synchronous enable and reset pins.

Figure A-28. The Schematic Version of the Behavioral BCD Counter

It’s also possible to use this single digit BCD counter with other copies of itself to build multi-digit BCD counters. The following is the listing of a complete 3-digit decimal counter.

entity BCD_CNTR is
  port (	BCD_UP_ENABLE	: in      STD_LOGIC;
        	BCD_CLK     	: in      STD_LOGIC;
        	BCD_RESET   	: in      STD_LOGIC;
        	BCD_QOUT    	: buffer  STD_LOGIC_VECTOR(11 downto 0));
end BCD_CNTR;

architecture BEHAV of BCD_CNTR is

component DECADE
  port (	UP_ENABLE	: in      STD_LOGIC;
        	CLK     	: in      STD_LOGIC;
        	RESET   	: in      STD_LOGIC;
        	QOUT    	: buffer  STD_LOGIC_VECTOR(3 downto 0);
        	COUNT_OUT	: out     STD_LOGIC);
  end component;

signal INTERNAL_ENABLE : STD_LOGIC_VECTOR(3 downto 0);

begin
  INTERNAL_ENABLE(0) <= BCD_UP_ENABLE;
    HUEY: DECADE
      port map (	UP_ENABLE	=> INTERNAL_ENABLE(0),
               	CLK     	=> BCD_CLK,
               	RESET   	=> BCD_RESET,
               	QOUT    	=> BCD_QOUT(3 downto 0),
               	COUNT_OUT	=> INTERNAL_ENABLE(1));
    DEWEY: DECADE
      port map (	UP_ENABLE	=> INTERNAL_ENABLE(1),
               	CLK     	=> BCD_CLK,
               	RESET   	=> BCD_RESET,
               	QOUT    	=> BCD_QOUT(7 downto 4),
               	COUNT_OUT	=> INTERNAL_ENABLE(2));
    LOUIE: DECADE
      port map (	UP_ENABLE	=> INTERNAL_ENABLE(2),
               	CLK     	=> BCD_CLK,
               	RESET   	=> BCD_RESET,
               	QOUT    	=> BCD_QOUT(11 downto 8),
               	COUNT_OUT	=> INTERNAL_ENABLE(3));
end BEHAV;

Figure A-29. Using a Single BCD Counter to Make a 3-Digit Decimal Counter

One concept being demonstrated here is how to import smaller designs in VHDL to build larger designs using the component statement. If you check, the component-end-component statement is just a rehash of Figure A-25, the port list of the single BCD counter. By including this component in the current design it allows you to copy it into the current design (called “instantiating.”) You have to give each instance of a component a unique name to distinguish it from other instances of the same design. In this case, we’ve used “HUEY”, “DEWEY”, and “LOUIE”. Another way of instantiating these three components would be:

signal INTERNAL_ENABLE : STD_LOGIC_VECTOR(3 downto 0);

begin
  INTERNAL_ENABLE(0) <= BCD_UP_ENABLE;
  DONALDS: for I in 1 to 3 generate
    DUCKS: DECADE
      port map (	UP_ENABLE	=> INTERNAL_ENABLE(I-1),
               	CLK     	=> BCD_CLK,
               	RESET   	=> BCD_RESET,
               	QOUT    	=> BCD_QOUT((I*4)-1 downto (I*4)-4),
               	COUNT_OUT	=> INTERNAL_ENABLE(I));
  end generate;
end BEHAV;

Figure A-30. Using a generate to Make a 3 Digit Decimal Counter

VHDL has a construct called generate that automatically makes as many copies of anything you specify. In this case, you came up with a clever way to use a generate to instantiate 3 copies of the “DECADE” component. generates must be labeled, in this case you use “DONALDS”. The component being copied (”DECADE”) must also have a naming tag, in this example “DUCKS.” The use of a generate in Figure A-30 is functionally equivalent to the bottom part of Figure A-29 -- and more compact to write!

Modeling Behavior in VHDL

The universal bi-directional shift register shifts “left” or “right” as its “mode” pins dictate. But, before you go into this design you should explore some of the basic differences between a signal and variable in VHDL designs.

Essentially a variable inside a VHDL design takes on whatever value that’s assigned to it immediately while a signal takes on what’s assigned to it after a infinitesimally small delay. This may not sound like something very important -- but it has huge implications.

entity KILROY is
  port (	VAR_IN, SIG_IN, CLK		: in   STD_LOGIC;
        	VAR_OUT, SIG_OUT   		: out  STD_LOGIC);
end KILROY;

architecture BEHAV of KILROY is
  signal X, Y, Z : STD_LOGIC;
begin
  process
    variable A, B, C, D : STD_LOGIC;
  begin
  wait until (CLK’EVENT and CLK = ’1’);

A := VAR_IN;  -- All signal assignments
    B := A;
    C := B;
    D := C;
    VAR_OUT := D;

X <= SIG_IN;  -- All variable assignments
    Y <= X;
    Z <= Y;
    SIG_OUT <= Z;

end process;
end BEHAV;

Figure A-31. Source Code Demonstrating the Fundamental Synthesis Differences between signals and variables

Because variables take on values assigned immediately, the variable part of “KILROY” synthesizes to a simple wire between “VAR_IN” and “VAR_OUT.” Because there’s an implicit delay in signals, synthesis builds a shift register for “SIG_IN” through “X”, “Y”, and “Z” to “SIG_OUT.”

Figure A-32. The Results of Synthesizing the VHDL in Figure A-31

Try analyzing the VHDL code listed below in Figure A-33 for this loadable 4-bit universal shifter as if you were a synthesis tool. Would your results look like Figure A-34’s? (Note “&” is not an “and” operator but a concatenation operator in VHDL!)

entity SHFT is
  port (	DATA_IN 	: in     STD_LOGIC_VECTOR(3 downto 0);
        	LF_SER_IN	: in     STD_LOGIC;
        	RT_SER_IN	: in     STD_LOGIC;
        	CLK     	: in     STD_LOGIC;
        	MODE    	: in     STD_LOGIC_VECTOR(1 downto 0);
        	CLEAR   	: in     STD_LOGIC;
        	QOUT    	: buffer STD_LOGIC_VECTOR(3 downto 0));
end SHFT;

architecture BEHAV of SHFT is
begin
  process begin
    wait until (CLK’EVENT and CLK = ’1’);
    if (CLEAR = ’1’) then
      QOUT <= ”00000000”;
    else
      case MODE is
        when ”00” => QOUT <= QOUT;  -- Change Nothing
        when ”01” =>
          QOUT <= QOUT(2 downto 0) & LF_SER_IN; -- Shift Left
        when ”10” =>
          QOUT <= RT_SER_IN & QOUT(3 downto 1); -- Shift Right
        when ”11” => QOUT <= DATA_IN;  -- Parallel Load Data_in
        when others => null;
      end case;
    end if;
  end process;
end BEHAV;

Figure A-33. Behavioral Code for a Loadable 4-bit Universal Shifter

Figure A-34. Conceptual Synthesis “Results” of a Loadable 4-bit Universal Shifter

Making a Multiplier

As a beginning text on VHDL, using a function to make a multiplier is a good way of learning how to do both. A quick way to multiply would be to use the “*” operator -- but that’s too easy!

Look first at the basic problem of multiplying to 4-bit numbers. This should quickly rekindle college memories of the old “shift-and-add” multiplication algorithm.

multiplicand: 1011

multiplier   x1001

-----

1011

0000-

0000-- *

1011--- *

-------

1100011

Figure A-35. The Basic 4-bit Multiplication Problem Using the “shift-and-add” Algorithm

As you can probably guess, if you’re not already familiar with the basic “shift-and-add” algorithm does use a lot of adds. Something where you can feed it two numbers and just get the sum (i.e. a function) would be nice to have. A sample adding function looks like:

function ADD ( A, B : in STD_LOGIC_VECTOR(3 downto 0))
  return STD_LOGIC_VECTOR(4 downto 0) is
    begin
      return ((’0’ & A) + (’0’ & B));
    end;

Figure A-36. A function to Add Two 4-bit Numbers with a Resulting 5-bit Sum

Every time a function is called, it’s fed only inputs and it creates a single output as its return value. Incorporated within the main design below (right under the architecture statement), the ADD function is used with the concatenation operator (”&”) to create the “shift-and-add” multiplication algorithm. Each call of a function causes a new return value to be calculated based on what it is fed.

entity MUL4 is
  port (	MP, MC	: in  STD_LOGIC_VECTOR(3 downto 0);
        	PRD   	: out STD_LOGIC_VECTOR(7 downto 0));
end MUL4;

architecture BEHAV of MUL4 is
  function ADD ( A, B : in STD_LOGIC_VECTOR(3 downto 0))
    return STD_LOGIC_VECTOR(4 downto 0) is
    begin
      return ((’0’ & A) + (’0’ & B));
    end;
  signal RS1, RS2, RS3 : STD_LOGIC_VECTOR(7 downto 0);
begin

RS3 <= (’0’ & MC & ”000”)
         when (MP(0) = ’1’)
         else ”00000000”;

RS2 <= (ADD(RS3(7 downto 4), MC) & RS3(3 downto 1))
         when (MP(1) = ’1’)
         else ’0’ & RS3(7 downto 1);

RS1 <= (ADD(RS2(7 downto 4), MC) & RS2(3 downto 1))
         when (MP(2) = ’1’)
         else ’0’ & RS2(7 downto 1);

PRD <= (ADD(RS1(7 downto 4), MC) & RS1(3 downto 1))
         when (MP(3) = ’1’)
         else ’0’ & RS1(7 downto 1);
end BEHAV;

Figure A-37. The “shift-and-add” Multiplication Algorithm in VHDL

NOTE:

Each signal is a stand alone piece of logic. Since there is no wait until or process with timing, this is purely a combinational logic design with “RS3”, “RS2”, “RS1”, and “PRD” rippling adds to get the product.

Figure A-38. The Schematic Representation of the “shift-and-add” Multiplication Algorithm from Figure A-37

While to some this algorithm will appear self evident, most people can just run a few test cases and see that yes, this does in fact work as a multiplier.

Designs Using Generics

Through the use of generics one can create generic designs in VHDL. This is basically one design that’s been generalized such that it can be customized on a “as needed” basis. For example, if you have an application where all you need is a 13-bit adder, it’s something of a waste to build a 16-bit adder.

Figure A-39. The Basic Building Block of Any Ripple Adder -- the 1-bit Full Adder

The equivalent VHDL code for building block looks like:

entity FULL_ADD is
  port (	A, B, CIN	: in   STD_LOGIC;
        	COUT, SUM	: out  STD_LOGIC);
end FULL_ADD;

architecture BEHAV of FULL_ADD is
begin
  SUM  <= (A xor B) xor CIN;
  COUT <= ((A xor B) and CIN) or (A and B);
end BEHAV;

Figure A-40. VHDL for A 1-bit Full Adder

In the following example seek out “WIDTH” and how it is used all throughout the design. The generic statement is used in one place to give “WIDTH” its value.

entity RIPPLE is
	generic (	WIDTH: INTEGER := 16);
	port (	NA, NB	: in  STD_LOGIC_VECTOR(XXX-1 downto 0);
        	NCIN  	: in  STD_LOGIC;
        	NSUM  	: out STD_LOGIC_VECTOR(XXX-1 downto 0);
        	NCOUT 	: out STD_LOGIC);
end RIPPLE;

architecture BEHAV of RIPPLE is

component FULL_ADD
    port (	A, B, CIN	: in   STD_LOGIC;
          	COUT, SUM	: out  STD_LOGIC);
  end component;

signal INTERNAL_WIRE : STD_LOGIC_VECTOR(WIDTH downto 0);

begin
  INTERNAL_WIRE(0) <= NCIN;

JECKLE: for I in 0 to XXX-1 generate
    HYDE: FULL_ADD
      port map (	A  	=> NA(I),
               	B  	=> NB(I),
               	CIN	=> INTERNAL_WIRE(I),
               	COUT	=> INTERNAL_WIRE(I+1),
               	SUM	=> NSUM(I));
  end generate;

NCOUT <= INTERNAL_WIRE(XXX);

end BEHAV;

Figure A-41. A Generalized N-bit Ripple Adder Using generics And 1-bit Full Adders

With the use of the generic construct, this generalized design could be used for anything from a 1-bit adder to a (2**32)-bit adder! (INTEGER data types default to 32-bits unless you specify a subrange that’s a lessor power of 2.)

Figure A-42. An N-bit Ripple Adder

To customize this current 16-bit adder in Figure A-41, all you would have to do is change the ‘16’ in the line ‘generic ( WIDTH : INTEGER := 16);’ to whatever value you desire.

Also, in Figure A-40 notice the use of parentheses on the equation for “COUT” to insure that the correct logic is built. It’s always wise to not assume any order of precedence or evaluation for VHDL operators.

Functions, Procedures, and Packages

Over time, as you write more and more VHDL, you’ll develop your own personalized library of designs, functions and procedures you may like using over and over again. In this example, we’ll build a 16-bit carry look-ahead adder using 4-bit parts from a package. (Remember, VHDL calls libraries packages.)

Figure A-43. Gate Diagram of a Carry Look-ahead Generator Plus Next Level Generate and Propagate Equations (GEN and PROP)

package LOOK_AHEAD is
	function CARRY_LA (	G, P	: STD_LOGIC_VECTOR(3 downto 0);
                    	CIN 	: STD_LOGIC);
    return STD_LOGIC_VECTOR(3 downto 0);
	procedure	GEN_PROP (G, P     	: in  STD_LOGIC_VECTOR(3 downto 0);
	        	     			GEN, PROP	: out STD_LOGIC);
end LOOK_AHEAD;

package body LOOK_AHEAD is

  function CARRY_LA (G, P	: in STD_LOGIC_VECTOR(3 downto 0);
                   CIN  	: in STD_LOGIC) is
    variable CR: STD_LOGIC_VECTOR(3 downto 0) := ”0000”;
	begin
		CR(0)	:= CIN;
		CR(1)	:= G(0)	or (CR(0) and P(0));
		CR(2)	:= G(1)	or (G(0) and P(1)) or (CR(0) and P(1) and P(0));
		CR(3)	:= G(2)	or (G(1) and P(2)) or (G(0) and P(2) and P(1))
                 	or (CR(0) and P(2) and P(1) and P(0));
    return CR;
  end;

procedure GEN_PROP(G, P      : in  STD_LOGIC_VECTOR(3 downto 0);
                     GEN, PROP : out STD_LOGIC) is
  begin
    GEN  <= G(3)
           	or (G(2) and P(3))
           	or (G(1) and P(3) and P(2))
           	or (G(0) and P(3) and P(2) and P(1));
    PROP <= P(3) and P(2) and P(1) and P(0);
  end;
end LOOK_AHEAD;

Figure A-44. VHDL of a Carry Look-ahead Generator with Next Level Generate and Propagate Equations (GEN and PROP)

Notice how the variable “CR” is initialized to “0000” within the “CARRY_LA” function. Also the function has no implicit registers or memory aspects built in by it’s sole use of variables; it’ll be pure combinational logic when it’s synthesized. The procedure achieves the same pure combinational logic form by having only internal equations using only signals and no wait’s nor processes. (By default, every in, out, inout, and buffer is a signal.) Also notice the use of parenthesis to insure the correct evaluation of the ANDs and ORs plus to hand structure the logic.

library		IEEE;
use			IEEE.STD_LOGIC_1164.all;
use			work.LOOK_AHEAD.all;

entity CLA is
  port (	A, B	: in  STD_LOGIC_VECTOR(3 downto 0);
        	CIN 	: in  STD_LOGIC;
        	SUM 	: out STD_LOGIC_VECTOR(3 downto 0);
        	GG  	: out STD_LOGIC;
        	GP  	: out STD_LOGIC);
end CLA;

architecture BEHAV of CLA is
  signal P : STD_LOGIC_VECTOR(3 downto 0);
  signal G : STD_LOGIC_VECTOR(3 downto 0);
  signal C : STD_LOGIC_VECTOR(3 downto 0);
begin

KILROY: for I in 0 to 3 generate
		P(I) 	<=	A(I)		xor	B(I);          	-- Propagate Equations
		G(I) 	<=	A(I)		and	B(I);          	-- Generate Equations
		SUM(I)	<=	(A(I)	xor	B(I)) xor C(I);	-- Sum Equations
  end generate;

C <= CARRY_LA(G, P, CIN);
									-- function call from work.LOOK_AHEAD.all
  GEN_PROP(G, P, GG, GP);
									-- procedure call from work.LOOK_AHEAD.all

end BEHAV;

Figure A-45. The Use of packages for Design Reuse in Building a 4-bit Carry Look-ahead Adder.

Near the top of this VHDL listing you’ll see “use work.LOOK_AHEAD.all;” which is where your custom packages are kept. Also, there was no need to include a component declaration because it’s all contained and accessible in your custom package. The basic design this VHDL makes is:

Figure A-46. A 4-bit Full Adder with Look-ahead Carry.

What follows is our final step in a complete example shown in a hierarchical design. Below a 16-bit carry look-ahead adder is built using four 4-bit adders from Figure A-45 which were, in turn, built from a package in shown in Figure A-44, which, in turn, relies on the IEEE.STD_LOGIC_1164 package for the definition of its nine-valued logic. If any of these parts were missing the design couldn’t be built. But, with design reuse, you don’t have to continuously redo designs you’ve done before.

entity GRP_CLA is
  port (	XA, XB	: in  STD_LOGIC_VECTOR(15 downto 0);
        	XCIN 	: in  STD_LOGIC;
        	XSUM 	: out STD_LOGIC_VECTOR(15 downto 0);
        	XG   	: out STD_LOGIC;
        	XP   	: out STD_LOGIC);
end GRP_CLA;

architecture BEHAV of GRP_CLA is
  component CLA
    port (	A, B	: in  STD_LOGIC_VECTOR(3 downto 0);
          	CIN	: in  STD_LOGIC;
          	SUM	: out STD_LOGIC_VECTOR(3 downto 0);
          	GG 	: out STD_LOGIC;
          	GP 	: out STD_LOGIC);
  end component;

  signal IC : STD_LOGIC_VECTOR(3 downto 0);
  signal IG : STD_LOGIC_VECTOR(3 downto 0);
  signal IP : STD_LOGIC_VECTOR(3 downto 0);

begin

  MARCO: for I in 1 to 4 generate
		POLLO: CLA port map (	A  	=>	XA((I*4)-1 downto (I*4)-4),
                        	B  	=>	XB((I*4)-1 downto (I*4)-4),
                        	SUM	=>	XSUM((I*4)-1 downto (I*4)-4),
                        	CIN	=>	IC(I-1),
                        	GG 	=>	IG(I-1),
                        	GP 	=>	IP(I-1));
  end generate;

  IC <= CARRY_LA(IG, IP, XCIN);

  GEN_PROP(IG, IP, XG, XP);

end BEHAV;

Figure A-47. A 16-bit Adder Built Using Four 4-bit Carry Look-ahead Adders

What Is It?

Having learned your VHDL skills from this quick tutorial, now’s a good time to test yourself. Below is a small design that’s described behaviorally in VHDL. What’s its function and what is it commonly called?

library		IEEE;
use			IEEE.STD_LOGIC_1164.all;

entity FOOBAR is
  port (	DATA	:	inout	STD_LOGIC_VECTOR(3 downto 0);
        	CLK 	:	in   	STD_LOGIC;
        	RDWR	:	in   	STD_LOGIC;        -- read ’1’, write ’0’
        	ADDR	:	in   	STD_LOGIC_VECTOR(2 downto 0));
end FOOBAR;

architecture BEHAV of FOOBAR is
	signal	DR : STD_LOGIC_VECTOR(3 downto 0);
	signal	R0, R1, R2, R3,
				R4, R5, R6, R7 : STD_LOGIC_VECTOR(3 downto 0);

begin
  process begin
    wait until (CLK’EVENT and CLK = ’1’);
    if (RDWR = ’0’) then
      case ADDR is
        when ”000” => R0 <= DATA;
        when ”001” => R1 <= DATA;
        when ”010” => R2 <= DATA;
        when ”011” => R3 <= DATA;
        when ”100” => R4 <= DATA;
        when ”101” => R5 <= DATA;
        when ”110” => R6 <= DATA;
        when ”111” => R7 <= DATA;
        when others => null;
      end case;
    end if;
  end process;

process (ADDR, R0, R1, R2, R3, R4, R5, R6, R7)
  begin
    case ADDR is
      when ”000” => DR <= R0;
      when ”001” => DR <= R1;
      when ”010” => DR <= R2;
      when ”011” => DR <= R3;
      when ”100” => DR <= R4;
      when ”101” => DR <= R5;
      when ”110” => DR <= R6;
      when ”111” => DR <= R7;
      when others => DR <= ”----”;
    end case;
  end process;

DATA <= DR when (RDWR = ’1’) else ”ZZZZ”;

end BEHAV;

Figure A-48. Mystery VHDL Design. What Is It? What Does It Do?

Note:

Answer is on the next page, written backward.

Answer:

.elif retsiger trop-elgnis sserdda 8 na s'tI