# VHDL Description Error Messages

Error & Warning Messages

The following errors and warnings indicate problems with the VHDL description. They may be generated during VHDL analysis and will appear on the screen, and in the .lis file. Execution is halted by the error, which must be corrected by modifying the VHDL description before continuing. Warnings will not halt the synthesis process, but should be noted and corrected if possible.

All messages are listed in numerical order, and explanations are given where needed for clarification.

0	name redefined at concurrent signal assignments

Explanation:  There is more than one concurrent assignment to the same target signal. Multiple assignment to the same signal in concurrent statement is not allowed.

1	 name not declared

Explanation:  Name must be declared before it can be used.

2	name redeclared

Explanation:  Multiple declaration is not allowed.

3	Name exceeds [#] characters

Explanation:  Name exceeds the number of characters shown.

4	IDENTIFIER expected

Explanation:  Syntax error.

5	bad identifier, possibly a reserved word

Explanation:  Syntax error.

6	Name expected

Explanation:  Syntax error.

7	Selected name: not implemented

Explanation:  The selected name is not implemented in this version.

8	SIGNAL expected

Explanation:  Expected the object to be a signal. This may be caused by a signal assignment to a non-signal object.

9	Vlbit value must be one of 0/1/X/Z

Explanation:  The constant token is not one of ‘0’, ‘1’, ‘X,’ or ‘Z.’

10	Support decimal literal index, others: not implemented

Explanation:  Only constant integer value is supported for indexing.

12	COMPONENT expected

Explanation:  Expected the object to be a component.

13	Error in sequential statement

Explanation:  Syntax error.

14	Error in IF-THEN statement

Explanation:  Syntax error.

15 	Only concatenate on signals, other cases: not implemented

Explanation:  Syntax error.

16	Boolean expression expected

Explanation:  The conditional expression used in IF statement must be a boolean value expression.

17	 Error in expression

Explanation:  Syntax error.

18 	CONVERSION function: not implemented

Explanation:  Conversion function used, but not supported in this version.

19		Support only signed decimal literals

20	Severity_level expected

Explanation:  WARNING. Expecting an enumerator literal of severity_level type.

21	Complex waveforms: not implemented

Explanation:  Complex waveforms used, but not supported in this version.

22	Error in wave form syntax

Explanation:  Syntax error.

23	Choices duplicated

Explanation:  The choices must be exclusive or unique.

24	Only use ‘0’/‘1’ in choices, other cases: not implemented

Explanation:  ‘X’ is not supported in choices.

25	Only need one OTHERS in choices

Explanation:  Multiple OTHERS not allowed.

26	OTHERS must be the only last choice

Explanation:  The choices after OTHERS are not allowed.

27	Discrete range in choice: not implemented

Explanation:  Range in choice used, but not supported in this version.

28	Error in CASE-WHEN choice

Explanation:  Syntax error.

29	Error in CASE-WHEN-ELSE of conditional waveform

Explanation:  Syntax error.

30 	Codegen failed on ENTITY <name> ARCHITECTURE: <name>

31	ENTITY name not found

Explanation:  Expecting the object to be an entity. This error may be caused if the corresponding entity declaration has not been successfully analyzed.

32	Design unit name failed

Explanation:  VHDM failed to analyze a design.

33	LIBRARY <name> not found or unsupported

Explanation:  Either the library is not installed, or has not been successfully analyzed.

34	PACKAGE name not found or unsupported

Explanation:  Either the package is not installed, or has not been successfully analyzed.

35	ARCHITECTURE name not found

Explanation:  Expecting the object to be an architecture. This error may be caused if the corresponding architecture declaration has not been successfully analyzed.

36	COMPONENT name not found

Explanation:  Expecting the object to be a component. This error may be caused if the corresponding component declaration has not been successfully analyzed.

37	Missing both Class & Mode, set to CONSTANT/IN

Explanation:  Neither class nor mode is specified for the object declaration. Set class to CONSTANT and mode to IN.

38	Error in signal declaration

Explanation:  Syntax error.

39	Error in interface declaration

Explanation:  Syntax error.

40	Only SIGNALs are allowed in port declaration

Explanation:  Non-signal objects found in port declaration.

41	Error in port interface list

Explanation:  Syntax error.

42	Only use component with port map, others: not implemented

Explanation:  Component instantiation without port map is not supported in this version.

43	Error in component declaration

Explanation:  Syntax error.

44	Checking port map aspect: BUS ignored

Explanation:  WARNING. “BUS” is not supported and is ignored.

45	Checking port map aspect: REGISTER ignored

Explanation:  WARNING. REGISTER is not supported and is ignored.

46	Too many actual parameters

Explanation:  More parameters are passed to the subprogram than the subprogram can accommodate.

47	Not enough actual parameters

Explanation:  Not enough parameters passed to the subprogram.

48	Unknown formal port I/O mode

Explanation:  VHDL failed to recognize the mode of the formal port.

49	Unknown actual port I/O mode

Explanation:  VHDL failed to recognize the mode of the actual port.

50	OUT actual to IN formal

Explanation:  Associating an output object to an input parameter is not allowed.

51	IN actual to OUT formal

Explanation:  Associating an input object to an output parameter is not allowed.

52	Non INOUT actual to INOUT formal

Explanation:  Associating a non-INOUT object to an INOUT parameter is not allowed.

53	Many actuals map to formal port name

Explanation:  Expecting a one-to-one association at the formal port.

54	No actuals map to formal port name

Explanation:  Expecting a one-to-one association at the formal port. Nothing is connected to the formal port.

55	Formal port name not found

Explanation:  The formal port is used in the port map association, but is not declared.

56	Reading an OUT port

Explanation:  WARNING. Attempting to read a port but the port’s mode is OUT.

57	Error to drive IN port

Explanation:  Attempting to drive a port but the port’s mode is IN.

58	Mixed name/position not allowed

Explanation:  Both named association and positional association are used.

59	Only support positional element association

Explanation:  Syntax error.

60	Labels do not match: name vs. name

Explanation:  The corresponding label and end-label must be the same.

61	name not in sensitivity list: FULL list required

Explanation:  A signal is used in the process, but is not included in the sensitivity list of the process.

62	Process has both sensitivity list & wait statement

Explanation:  A process cannot have both a sensitivity list and wait statement.

63	Process has no sensitivity list & no wait statement

Explanation:  A process must have either a sensitivity list or a wait statement. A process without a sensitivity list or wait statement represents an infinite loop process, and is not supported in synthesis.

64	Error in process sensitivity list/declaration

Explanation:  Syntax error.

65	Error in process sensitivity list

Explanation:  Syntax error.

66	Explicit sensitivity list on WAIT: not implemented

Explanation:  Explicit sensitivity list used, but not supported.

67 	Only support ‘WAIT’ as the first statement, others
		 not implemented

68 	Only support ‘WAIT UNTIL condition’ others not implemented

Explanation:  The WAIT statement does not conform to the synthesis style.

69	Timeout on WAIT not implemented

Explanation:  WARNING. Timeout on WAIT used but not supported.

70	Need a prising/pfalling on WAIT statement, others: 
		not implemented

Explanation:  Expecting prising or pfalling of a clock signal on the WAIT statement.

71	Need only one prising/pfalling on a 1-bit signal,
		others: not implemented

Explanation:  More than one clock signal is specified. Only one clock signal is supported for a clocked process.

72	Only support prising/pfalling on 1-bit signal, others: 
		not implemented

Explanation:  The clock specified is not a single bit signal.

73	Conditions on WAIT statement may only be ORed

Explanation:  Only OR operators are supported in the conditional expression of the WAIT statement. This is to OR clock event and asynchronous set/rest conditions for the WAIT statement.

74	Conditions on WAIT must be an equality on a signal

Explanation:  Either a non-signal object was used or an operator other than equality was used on a signal.

75		Data with [#] bits, need [#] bits

Explanation:  Wrong number of bits specified.

76	Unknown constant type in initialization

Explanation:  A constant is specified in initialization, but its type cannot be determined.

77	Deferred constant: not implemented

Explanation:  Deferred constant declaration in package is not supported.

78	Error in constant declaration

Explanation:  Syntax error.

79	vector vs. logic vector type in initialization

Explanation:  Type of constant value does not match the type of the constant object.

80		bit vs. vector in initialization

Explanation:  Type of constant value does not match the type of the constant object.

81	initialization ignored in signal/process variable declaration

Explanation:  Initial value specified in signal is ignored. Initial value specified in variable declaration in a process is ignored.

82	Types do not match, or size error

Explanation:  The object used is the wrong type or wrong size.

83	bit/vector vs. BIT_STRING

Explanation:  Type does not match.

84	vector expected

Explanation:  The object is not logic vector type.

85	bit expected

Explanation:  The object is not logic type.

86	vector expected not BIT_STRING

Explanation:  WARNING. Expecting logic vector object.

87	bit vs. vector are different types

Explanation:  Type does not match.

88	bit is not an array subtype

Explanation:  Type does not match.

89	Only support bit & vector without constraints

Explanation:  The object type is not supported.

90	Only support bit & vector without resolution

Explanation:  Resolution function used, but not supported.

91	Null/unconstrained array: not implemented

Explanation:  Null array or unconstrained array used, but not supported.

92	Only handle 1-d array of scalars, others:
		not implemented

Explanation:  Multi-dimensional array used, but not supported.

93	Bad direction and becomes a null slice

Explanation:  The array index of the object is declared in one direction but is used in the opposite direction. For example, the object is declared to be “a(o to 5)” but is used as 
“a (5 downto 0)”.

94	Index out of range

Explanation:  The value of the index is outside of the declared range of the array object.

95	Error in index constraint

Explanation:  Syntax error.

96 	Unexpected token in ENTITY header

Explanation:  Syntax error.

97	Missing keywords IS an ENTITY header

Explanation:  Syntax error.

98	ENTITY identifier expected

Explanation:  Syntax error.

99	ENTITY name not match: name vs. name

Explanation:  Entity name does not match the name in the entity declaration.

100	COMPONENT declaration not allowed in ENTITY

Explanation:  You are declaring a component in an entity. This is not allowed in VHDL.

101	Error in entity statement

Explanation:  Syntax error.

102	Error in entity declaration

Explanation:  Syntax error.

103	Only support ‘use lib.pkg.ALL;’ others: not implemented

Explanation:  The USE statement, other than the form of ‘use lib.pkg.ALL,’ is not supported.

104	PACKAGE name not match: name vs. name

Explanation:  Package name does not match the package name declared.

105	Error in package header

Explanation:  Syntax error.

106	Error in package declaration

Explanation:  Syntax error.

107	SIGNAL declaration in PACKAGE: not implemented

Explanation:  You have declared a signal in a package. This is not supported.

109	Error in package body header

Explanation:  Syntax error.

110	ARCHITECTURE name not match: name vs. name

Explanation:  Architecture name does not match the architecture name declared.

111	Error in architecture header

Explanation:  Syntax error.

112	Error in declarations in architecture body

Explanation:  Syntax error.

113	Error in concurrent statement

Explanation:  Syntax error.

114	Error in port map aspect

Explanation:  There is a syntax error in the port map of a component instantiation statement.

115	Top level conditions should be one of the conditions in WAIT

Explanation:  The out-most conditions of the IF THEN ELSE statement must be one of the conditions in the WAIT statement.

116	Some asynchronous set/reset conditions in WAIT not used

Explanation:  Some of the asynchronous set/reset conditions in the WAIT statement of the process are not used as the out-most condition. You specified set/reset conditions, but did not use them.

117		Statement must be in the conditions in WAIT

Explanation:  Unconditional assignment statement is not allowed in a process that has set or reset conditions.

118	Asynchronous set/reset blocks should follow clocking block

Explanation:  The asynchronous set or reset condition has lower priority than the clocking condition. This cannot be implemented.

119	Only allow prising/pfalling on WAIT/IF statement

Explanation:  Functions other than prising or pfalling are not allowed in a WAIT statement.

120	IF prising/pfalling must follow WAIT statement immediately

Explanation:  The clocking condition has higher priority than the asynchronous set or reset condition. This cannot be implemented.

121		NO ELSE/ELSIF is allowed in this context

Explanation:  Error in the syntheses style for clocked process. This cannot be implemented.

122		Please use ‘USE synth stdsynth ALL’

Explanation:  You should use vlbit/vlbit_1d types.

123	Bad identifier too many ‘_’

Explanation:  There are more than one consecutive underscores (“_”) in this identifier.

124	Only SIGNALs/OPEN are allowed in port map

Explanation:  You are using expressions other than signals or OPEN in a component instantiation port map.

125		Bad operator symbol; see Reference Manual for operators

Explanation:  Illegal operator used.

126	Don’t care ‘X’ can not be used with logical operators

Explanation:  ‘X’ is used in the expression, but is not supported.

127		Don‘t care ‘X’ in waveform not implemented

128		Multiple drivers to signal <name> (Driver at line <line>)

Explanation:  ‘X’ is used in the expression, but is not supported.

129		BLOX <name> not supported

130		Default expression required for OPEN association to input port

Explanation:  The input port of the component is OPEN, but no default value has been specified for this port.

131		TIME expression: not checked

Explanation:  WARNING. The time expression is ignored.

132	Incomplete vhdl source file

Explanation:  Syntax error at the end of the VHDL source file.

134		Integer to logic vector type conversion overflow, higher bits lost

Explanation:  An integer value has been converted to a logic vector object, but the object is not big enough to hold the entire integer value.

For example, 11 is converted to “1011” but the target object has three bits. Only “011” is converted.

135		GUARDED option unsupported

Explanation:  GUARDED mode used, but not supported.

136		TRANSPORT option unsupported

Explanation:  TRANSPORT mode used, but not supported.

137		LINKAGE mode unsupported

Explanation:  LINKAGE mode used, but not supported.

138	IN actual must associate with IN formal

Explanation:  The formal parameter has mode IN. Therefore, the actual parameter’s mode must also be IN.

139	OUT actual must associate with OUT formal

Explanation:  The formal parameter has mode OUT. Therefore, the actual parameter’s mode must also be OUT.

140		Library clause not used for this ‘use clause’

Explanation:  WARNING. A “USE lib.pkg.ALL” statement is used with no corresponding “library lib;” statement.

141		Codegen failed on SUBPROGRAM <name>

142	Uplevel reference in FUNCTION: not allowed

Explanation:  Reference of a signal or a variable outside of a FUNCTION is not supported.

143	Nested subprogram: not supported

Explanation:  Subprogram references from within a subprogram are not supported.

144		Subprogram in process: not supported

Explanation:  Declaration of subprogram header is not supported in process statement.

145	Subprogram overloading: not supported

Explanation:  Subprograms that have the same name are not supported.

146	Empty parameter list: not supported

Explanation:  Subprograms that have no parameters are not supported.

147	Support one outermost loop only for procedure

Explanation:  More than one loop is used, or the loop is not the outermost loop.

148		WAIT must be the first statement in the loop of PROCEDURE

Explanation:  WAIT statement must be the first statement in the outermost loop of a procedure. The WAIT statement is missing.

149	Only allow one RETURN as the last statement in a FUNCTION

Explanation:  More than one RETURN statement used, or the RETURN statement is not the last statement in the function body.

150	A function must RETURN an expression as the last statement

Explanation:  A RETURN statement is missing, and is required in the function.

151	CONFIGURATION: not implemented

Explanation:  WARNING. CONFIGURATION used by not implemented.

152		Support only stand-alone function call, i.e. 1hs <= fun_call

153	Variable <name> in subprogram initialized to 0’s

Explanation:  Variable declared in a subprogram has no initial value. The initial value is set to ‘0.’

154		Subprogram declaration not supported in PACKAGE BODY

155		Name reserved for DXP primitive

156		VARIABLE declaration only allowed in PROCESS and 
		subprogram

Explanation:  A variable is declared, but this is not allowed here. A variable can only be declared in a PROCESS or a subprogram.

157	PROCEDURE/FUNCTION name not match:
		name vs. name

Explanation:  The end name does not match the declared name in a subprogram declaration.

160		IN mode expected for GENERIC/FUNCTION formals

Explanation:  The mode of the formal parameters in GENERIC or FUNCTION must be IN.

161	Only use IN/INOUT/OUT mode for PROCEDURE formals

Explanation:  Modes other than IN/INOUT/OUT have been specified for PROCEDURE parameters.

162	Subtype not supported for this object

Explanation:  This subtype is not supported.

163	Error in initial value

Explanation:  Syntax error.

164		Only support GENERIC formal with initial value

Explanation:  Initial value is not specified.

165	Named association: not implemented

Explanation:  Named association used, but not implemented.

166	Only support TIME/INTEGER literal value for GENERIC actual

Explanation:  Generic parameters other than TIME or INTEGER types have been used.

167	Component <name> and previously analyzed
		entity have different ports

Explanation:  The ports do not match.

168		Generic formal not in ENTITY

Explanation:  Generic parameter used in component, but not in the corresponding entity declaration.

169	Only support subprograms in ENTITY, ARCHITECTURE 
		and PACKAGE

Explanation:  Subprogram not allowed here.

170	Potential error for generically constrained object size

Explanation:  The object size is generically constrained and may be changed by the actual generic value passed in.

171	Output <name>  has no driver

Explanation:  There is no signal assignment to this output port.

172		Only support range using DECIMAL LITERALS

Explanation:

173	Decimal literal expected

Explanation:  Expecting the current token to be a decimal constant.

174	Null range: not supported

Explanation:  The range specified is a null range.

175	Attribute designator expected

Explanation:  The object is not declared as an attribute.

176		User-defined attributes ignored

Explanation:  User-defined attributes are not supported.

177	Attribute ignored: only support synthesis attributes in ENTITY

Explanation:  User attribute specification is not supported.

178	This attribute applies only to input port

179		This attribute applies only to output port

180		Aggregate of two decimal literals expected

181		Boolean literal expected

182		Only one decimal literal expected

183		Attribute <name> not supported

184		Attribute <name> re-specified for <name>

185		Attribute <name> not supported for <name>

186		Object <name> not of class <name>

187		Character literal expected

Explanation:  Expecting the current token to be a character value.

188		Found ‘sequential statement’; ‘concurrent statement’ expected

Explanation:  Sequential statement is not allowed here.

189	Found ‘concurrent statement’; ‘sequential statement’ expected

Explanation:  Concurrent statement is not allowed here.

190	No input formals for this design

Explanation:  No input parameters are specified.

191	No output formals for this design

Explanation:  No output parameters are specified.

192		Input <name> has no load

Explanation:  This input is not used.

193		2-dimensional object needs two indices

194	Divide by zero

Explanation:  Attempting to divide by zero.

195	Boolean constant expression expected

Explanation:  The value of the expression is not of Boolean type.

196	Constant expression expected

Explanation:  The expression is not a constant value.

197		Generate label required

Explanation:  Missing label for GENERATE statement.

198	Generate loops > 10 levels

Explanation:  Too many levels.

199	Array object/subtype expected

Explanation:  The range or reverse-range attribute requires an array object.

200	Only support ‘integer range <>’ in unconstrained array

Explanation:  Unconstrained array is specified, but the specified range is not “integer 
range <>.”

202	Parameter error with subprogram call

Explanation:  Syntax error.

203	Non-negative integer value expected

Explanation:  Negative integer value used, positive integer value expected.

204	Bit vector object expected

Explanation:  Expecting a bit vector object.

205		Aggregate target: not implemented

Explanation:  Assignment to an aggregate object made, but not implemented.

206	Variable expected

Explanation:  Variable object expected. This may be caused by a variable assignment to a non-variable object.

207	Variable in process must be unconditionally initialized

Explanation:  The variable is conditionally initialized in a combinational process.

208	Operand truncated

Explanation:  The target object is smaller than the value assigned to it. The value is truncated.

209		NULL expression: not supported

Explanation:  NULL expression used, but not supported.

210	Only allowed to call this procedure a Concurrent Statement

Explanation:  Use of this procedure in a sequential statement is not supported.

211	Procedure body with uplevel reference must be in PROCESS

Explanation:  The procedure refers to a global signal or variable. This is only allowed if the procedure is declared in a process.

212	Procedural call in function body: not supported

Explanation:  To call a procedure form within a function is not supported.

213		Enumeration literal not found

Explanation:  The object is not declared as an enumeration type.

214	Second operand for DIV/MOD/REM must be power of 2

Explanation:  Expecting the second operand to be a constant that is a power of 2 value, such as “1, 2, 4, 8, 16...”

215	 ‘Z’ value supported only in conditional signal assignment

Explanation:  The tristate value ‘Z’ can only be used in concurrent signal assignment statements.

216	Only support simple name for ENTITY

Explanation:  Selected name not supported.

217		Filename not same as Entity name

Explanation:  WARNING. The file name is not the same as the entity name.

218	<name> is a registered variable

Explanation:  The variable was not unconditionally assigned before it was used in a clocked process. A register is allocated to store this variable.

219	Size error in PLA_TABLE ([#] bits + [#] bits /= [#] bits)

Explanation:  The PLA_TABLE has three parameters. The size of the third parameter must be equal to the sum of the first and second parameters.

220	Third actual of the PLA_TABLE must be a constant object

Explanation:  The third parameter of a PLA_TABLE must be a logic_2d constant.

221		Only allow procedure <name> in concurrent statement

Explanation:  This procedure is used in a sequential statement.

222	Bus name conflict on <name>

Explanation:  The term “bus” here means an “array” or a “vector.” Integer numbers are individual expanded names for the array port. Name collision has occurred during this name expansion.

223	Constraint error on 2-dimensional constant

Explanation:  The 2-dimensional constant does not conform to the object declared.

224	Illegal or unsupported operation

Explanation:  Operation used is not supported or is illegal.

225		Sub-element sensitivity used in <name> full sensitivity assumed

Explanation:  WARNING. Sub-element of an array signal used in the sensitivity list of a processes. It is assumed that the process is sensitive to all bus elements of the signal.

226	Subprogram <name> body must be defined before it can be called

Explanation:  A subprogram is called before the subprogram body is defined.

227	<library.pkg.name> selected but <library.pkg.ALL> assumed

Explanation:  An object is selected from a package, but ALL objects are assumed to be visible.

228		Please use type qualifier to resolve OVERLOADING of 
		enumeration

229		EBC Error

230		Operation not allowed in this context

231		Data with <#> bits, truncated to <#> bits

232		<name> is not a valid architecture

233		Only string attributes supported

234		Attribute <name> is not defined

235		Multiple declarations for attribute <name>