xilinx-specific synthesis

Chapter 1:    Designing with xilinx x-blox

Essential Requirements

Reference Material

Design Flow

Setting Up Your Environment

Configuring the viewdraw.ini File

viewdraw.ini File Configuration

X-BLOX Elements

ViewSynthesis for Windows Supported X-BLOX Modules

Design Examples

Behavioral VHDL Description

Pure VHDL Example: Counter/7-Segment Assembly

Combined VHDL/X-BLOX Description

Using X-BLOX within VHDL

VHDL/X-BLOX Example: Counter/7-Segment Assembly

Chapter 2:    specifying flip-flops with clock enable	10

Introduction

Basic VHDL Clock Enable Syntax Rules

General Formats

Annotated VHDL Example

Chapter 3:    automatic i/o pad insertion	13

Introduction

Invoking I/O Pad Mapping and Insertion

Types of Transformations

Complex I/O Mapping

Bi-directional Mapping

Clock Buffering

Default Pads

Specifying I/O Pad Locations

Chapter 4:    clb packing	16

Introduction

Invoking Technology Specific Mapping

CLB and LUT Basics

The Report File

XC4000 Reports

XC2000 and XC3000 Reports

Chapter 5:    passing timing constraints	19

Full Compatibility with Xilinx Tools

Including Timing Data in WIR File

appendix a:    x-blox procedures	20

Introduction

The X-BLOX VHDL Package

Passing Inputs, Outputs and Attributes to X-BLOX

Example:  Using a Named Association Procedure Call

Example: Using a Positional Procedure Call

Attribute Syntax and Usage

X-BLOX VHDL Package Listing

Overview

Essential Requirements

This version of ViewSynthesis for Windows has been shipped with all the synthesis technology files needed for the supported Xilinx families. Synthesis access of X-BLOX technology files is automatic through the WDIR environment directory list.

In addition to the synthesis technology files, you will need the X-BLOX module library and the Xilinx family symbol library from Xilinx, Inc. The X-BLOX module library provides variable-size MSI and LSI level building blocks (e.g., adders, counters, clock dividers, decoders, universal shift-registers, SRAM, PROM, and more), while the symbols provide the simpler, fixed-size logic and gate functions that completely define a circuit.

Reference Material

IMPORTANT

The X-BLOXTM Design Tool User Guide, from Xilinx, Inc., is a necessary adjunct to this manual.

Design Flow

A synthesized XC4000 design begins with a combination of four elements:

- your VHDL design

- an XC4000 symbol library

- an X-BLOX Module symbol library

- ViewSynthesis for Windows XC4000 technology data files

These elements are incorporated into Viewlogic’s ViewSynthesis for Windows tool that synthesizes the design to produce an optimized netlist called a “wirelist.”

This wirelist is handed off to a set of Xilinx, Inc. tools that convert it into Xilinx Netlist Format (XNF), perform X-BLOX module synthesis, then convert it back to Viewlogic tool-readable format.

The resulting wirelist can then be further processed by Viewlogic’s set of simulation design tools.

Note:

Refer to the X-BLOXTM Design Tool User Guide for detailed Xilinx design flow information.

Simulation Output

Figure 1-1. ViewSynthesis for Windows/X-BLOX Design Flow

Setting Up Your Environment

### Configuring the viewdraw.ini File

Your viewdraw.ini file should be configured as shown in Figure 1-2. Configuring the viewdraw.ini file as shown allows ViewSynthesis for Windows to find the XC4000 symbols and X-BLOX modules needed to synthesize your design.

In Figure 1-2 the:

- XC4000 symbols are in the xc4000 directory

- Xilinx X-BLOX modules are in the xblox directory

Note:

The XC4000 technology data files are pointed to in the WDIR environmental variable.

### viewdraw.ini File Configuration

The following example show how to configure your viewdraw.ini file when using either Xilinx’s XC4000 library.

DIR[p].
DIR[m]c:\xact\lib\xc4000 (xc4000)
DIR[m]c:\xact\lib\xblox (xblox)
DIR[m]c:\wvoffice\lib\builtin (builtin)
DIR[m]c:\wvoffice\lib\xbuiltin (xbuiltin)

Figure 1-2. XC4000 viewdraw.ini Configuration

IMPORTANT

All “(alias)” assignments are required.

X-BLOX Elements

### ViewSynthesis for Windows Supported X-BLOX Modules

The following list contains the X-BLOX modules that are supported by ViewSynthesis for Windows VHDL procedural calls.

| X-BLOX Module Name |  |
| --- | --- |
| ACCUM | INVBUS |
| ADD_SUB | MUXBUS |
| ANDBUS | MUXBUS2 |
| ANDBUS1 | MUXBUS4 |
| ANDBUS2 | MUXBUS8 |
| BIDIR_IO | ORBUS |
| BUS_DEF | ORBUS1 |
| CLK_DIV | ORBUS2 |
| COMPARE | OUTELEM |
| COUNTER | OUTPUTS |
| DATA_REG | PROM |
| DECODE | SHIFT |
| ELEMENT | SLICE |
| FORCE | SRAM |
| INC_DEC | TRISTATE |
| INELEM | XORBUS |
| INPUTS |  |

The following list contains the X-BLOX modules utilized by ViewSynthesis for Windows operator inferencing capability:

| ADD_SUB | COMPARE |
| --- | --- |
| INC_DEC |  |

Design Examples

Describing arithmetic components using normal, behavioral VHDL usually results in efficient circuit results targeting a highly efficient mix of X-BLOX components and “random logic.” However, by using X-BLOX procedures within a VHDL description, the you may exert  more control over the Synthesizer, and also target X-BLOX counters, which cannot be automatically inferred.

This section presents two examples of:

- A BCD down counter and 7-Segment display driver using behavioral VHDL

- The same design using the X-BLOX COUNTER procedure

SEG

Figure 1-3. Display Driver Assembly

### Behavioral VHDL Description

The following VHDL example describes an assembly consisting of a cascadable decimal down counter with outputs encoded as 7-segment drivers.

The first process builds a binary counter. The result is fed into a second process which translates it into 7-segment encoding.

A concurrent signal assignment is used to drive COUT which can be used to drive the next stage’s DN_ENABLE.

### Pure VHDL Example: Counter/7-Segment Assembly

library		SYNTH;
use			SYNTH.STDSYNTH.all;

entity CNT7SEGV is
	port (
		signal RESET, DN_ENABLE, CLK  : in  BIT;
		signal COUT : out BIT;
		signal SEG  : out BIT_VECTOR(6 downto 0));
end CNT7SEGV;

architecture ONE of CNT7SEGV is
	signal   BCD : BIT_VECTOR(3 downto 0);

-- The following process is replaced by an X-BLOX Counter module
-- in the next VHDL design example.

begin
		process 
			begin
			wait until (CLK’EVENT and CLK = ’1’ or (RESET=’1’));
			if RESET=’1’ then BCD <= ”1001”;
			else
				if (DN_ENABLE = ’1’) then BCD <= ”1001”;
				else BCD <= BCD - 1;
				end if;
			end if;
		end process;

begin
	process 
	begin
		SEG <= ”10011111”; -- Default error initialization
		case BCD is
			when ”0000” => SEG <= ”1111110”;
			when ”0001” => SEG <= ”0110000”;
			when ”0010” => SEG <= ”1101101”;
			when ”0011” => SEG <= ”1111001”;
			when ”0100” => SEG <= ”0110011”;
			when ”0101” => SEG <= ”1011011”;
			when ”0110” => SEG <= ”1011111”;
			when ”0111” => SEG <= ”1110000”;
			when ”1000” => SEG <= ”1111111”;
			when ”1001” => SEG <= ”1111011”;
			when other  => SEG <= ”1001111”;
		end case;
	end process;

COUT <= ’1’ when BCD = ”0000” else ’0’;
end ONE;

This behavioral description implements the statement BCD <= BCD - 1 with and INC_DEC X-BLOX module.

### Combined VHDL/X-BLOX Description

The VHDL description listed in this section uses an X-BLOX COUNTER process to define the counter portion of the binary counter/7-segment assembly shown in Figure 1-4.

Figure 1-4. Entity CNT7SEGX

### Using X-BLOX within VHDL

To use X-BLOX procedures in your VHDL design you must first declare the Xilinx library and X-BLOX package at the beginning of the VHDL description as shown below.

library		XILINX;
use			XILINX.XBLOX.all;

Use a named (or positional) association concurrent procedure call to assign the X-BLOCK inputs, outputs, and attributes as shown below.

xb_COUNTER (ASYNC_CTRL => RESET,
					 SYNC_CTRL  => COUT,
					 CLOCK      => CLK,
					 CLK_EN     => DN_ENABLE,
					 Q_OUT      => BCD,
					 UP_DN      => ’0’,
					 ATTR       => ”STYLE=BINARY”);

IMPORTANT

To use an X-BLOX process within VHDL you must precede the X-BLOX module name with “xb_” as shown in the example above: “xb_COUNTER.” 

Refer to the Xilinx X-BLOX Design Tool User Guide for complete attribute choices and additional usage information.

### VHDL/X-BLOX Example: Counter/7-Segment Assembly

library		XILINX;
use			XILINX.XBLOX.all;

library		SYNTH;
use			SYNTH.STDSYNTH.all;

entity CNT7SEGX is
	port (
		signal RESET, DN_ENABLE, CLK  : in  BIT;
		signal COUT : buffer BIT;
		signal SEG  : out BIT_VECTOR(6 downto 0));
end CNT7SEGX;

architecture ONE of CNT7SEGX is
	signal   BCD : BIT_VECTOR(3 downto 0);

-- The following X-BLOX counter procedure started out as a
-- clocked, behavioral decrement counter in CNT7SEGV.VHD.

xb_COUNTER (ASYNC_CTRL => RESET,
					 SYNC_CTRL  => COUT,
					 CLOCK      => CLK,
					 CLK_EN     => DN_ENABLE,
					 Q_OUT      => BCD,
					 UP_DN      => ’0’,
					 ATTR       => ”STYLE=BINARY”);

begin
	process 
	begin
		SEG <= ”10011111”; -- Default error initialization
		case BCD is
			when ”0000” => SEG <= ”1111110”;
			when ”0001” => SEG <= ”0110000”;
			when ”0010” => SEG <= ”1101101”;
			when ”0011” => SEG <= ”1111001”;
			when ”0100” => SEG <= ”0110011”;
			when ”0101” => SEG <= ”1011011”;
			when ”0110” => SEG <= ”1011111”;
			when ”0111” => SEG <= ”1110000”;
			when ”1000” => SEG <= ”1111111”;
			when ”1001” => SEG <= ”1111011”;
			when other  => SEG <= ”1001111”;
		end case;
	end process;

COUT <= ’1’ when BCD = ”0000” else ’0’;
end ONE;

After Xilinx PP&R, the behavioral design used:
	7 Packed CLBs
	46 ns worst case path delay

After Xilinx PP&R, the X-BLOX counter procedure used:
	5 Packed CLBs
	38 ns worst case path delay

Overview

Introduction

ViewSynthesis for Windows uses flip-flops with clock enable for Supported and applicable Xilinx technologies. Using flip-flops with built-in clock enable reduces your design’s component count, and achieves a faster performing circuit. ViewSynthesis for Windows derives the clock-enable automatically using flow analysis, but by using the templates in this manual, you will increase the likelihood that the synthesizer will recognize useful clock enable situations.

Note:

There are certain circumstances where ViewSynthesis for Windows does not use clock enable because it has computed that simply computing the “D” input is more efficient than computing both a “D” and a clock enable function.

Figure 2-1. Flip-flop with Clock Enable

Basic VHDL Clock Enable Syntax Rules

Required:

- The clock enable specification must be exclusively encapsulated within the synchronous section of a clocked process.

Optional:

- The clock enable specification should be the outermost if...then...else of the synchronous section of a clocked process.

General Formats

The following four descriptions all imply clock enable. Two also have asynchronous operations. We show the two major styles of register inferencing.

Wait Format without Asynchronous Reset Condition:

process
begin
	wait until <CLOCK_CONDITION>;
	if (<ENABLE_CONDITION>) then
		 <SIGNAL_ASSIGNMENTS>;
	end if;
end process;

Wait Format with Asynchronous Reset Condition:

process
begin
	wait until <CLOCK_CONDITION> or (<RESET_SIGNAL> = <BIT_CONST>);
	if (<RESET_SIGNAL> = <BIT_CONST>) then
		 <RESET_ASSIGNMENTs>;
	else
		if (<ENABLE_CONDITION>) then
			<SIGNAL_ASSIGNMENTS>;
		end if;
	end if;
end process;

Alternate Register Inference Format without Asynchronous Reset Condition:

process (<CLOCK_SIGNAL>)
begin
	if (<CLOCK_CONDITION>) then
		if (<ENABLE_CONDITION>) then
			<SIGNAL_ASSIGNMENTS>;
		end if;
	end if;
end process;

Alternate Register Inference Format with Asynchronous Reset Condition:

process (<CLOCK_SIGNAL>,<RESET_SIGNAL>)
begin
	if (<RESET_SIGNAL> = <BIT_CONST>) then
		 <RESET_ASSIGNMENTs>;
	elsif (<CLOCK_CONDITION>) then
		if (<ENABLE_CONDITION>) then
			<SIGNAL_ASSIGNMENTS>;
		end if;
	end if;
end process;

Annotated VHDL Example

The following annotated VHDL description defines a circuit that has two flip-flops with a common clock enable input. The circuit ViewSynthesis for Windows creates when using this VHDL description is shown in Figure 2-2.

entity ENB1 is
	port (
		RESET, CLK, DA, DB, ENABLE	:	in  BIT;
		QOUT, ZOUT:								out BIT);
end ENB1;

architecture ONE of ENB1 is
begin
	--
	-- Other processes and VHDL constructs 
	--
	-- The clock enable routine MUST be encapsulated within a 
	-- process.
	process  -- This must be a clocked process.
	begin
		wait until (CLK’EVENT and CLK = ’1’) or RESET = ’1’;
		--can be with or without reset
		if (RESET = ’1’) then
			QOUT <= ’1’;
			-- left hand notation (”QOUT”) is a signal (port or other)
			ZOUT <= ’0’;
		else
			if (ENABLE = ’1’) then
				QOUT <= DA;
				ZOUT <= not DB;
			end if;
		end if;
	end process;

	-- Other processes and VHDL constructs
end ONE;

Figure 2-2. ENB1 - Created by ViewSynthesis for Windows Using the Annotated VHDL Example

Overview

Introduction

When you synthesize to Xilinx technology families, you can direct ViewSynthesis for Windows to automatically add I/O pads to the design. This section describes how to invoke this feature, and how it operates.

Invoking I/O Pad Mapping and Insertion

ViewSynthesis for Windows modifies the wirefile created during the synthesis process to add I/O pads. You invoke this function using ViewSynthesis for Windows. In the Synthesis options dialog box, push the Tech Options button, and select the Insert I/O Pads option.

Types of Transformations

This section describes the steps taken by ViewSynthesis for Windows to add I/O pads to your design. The processing occurs in three parts. By understanding these parts, you will understand the modifications that the tool makes to your design.

First, “complex I/O mapping” takes place. During this phase, the tool finds any places where flip-flops or tri-states are used at primary inputs and outputs. Depending on what I/O macros are available in the technology library, these may be changed from “internal” flip-flops and tri-states into “pad” flip-flops and tri-states. For bi-directional ports, some additional processing is performed to make sure the circuit is correct.

Second, the tool propagates all signals which drive the clock input of latches or registers and assigns technology-specific clock drivers automatically.

Third, any input or output ports which have not been mapped to complex I/O or clock buffers are processed by inserting default buffers.

### Complex I/O Mapping

This section describes how mapping to pad flip-flops, latches and tri-states is performed. Most FPGA technologies provide tri-state drivers in the I/O ring, which must be used when an off-chip tri-state driver is needed. Many FPGA technologies provide programmable flip-flops or latches in the I/O ring. Using these elements, where available, improves circuit performance and density.

If the appropriate parts are available in the technology library you are using, the following types of transformations are made to the design.

- “Internal” flip-flops or latches at primary inputs or outputs are transformed into their “pad” equivalents. For example, in the Xilinx XC4000 family, an FDCE internal flip-flop at a primary input is transformed into an IFD input flip-flop.

- Tri-state drivers at primary outputs are transformed into their “pad” equivalents. For example, in the Xilinx XC4000 family, a TBUF internal tri-state at a primary output is transformed into an OBUFT pad tri-state.

- The combination of an internal flip-flop followed by a tri-state, at a primary output, is mapped into a pad ring equivalent. For example, in the Xilinx XC4000 family, an FDCE internal flip-flop, which drives a TBUF internal tri-state at a primary output is transformed into an OFDT pad cell, which contains both a flip-flop and a tri-state.

### Bi-directional Mapping

When a tri-state port is found, some additional processing is required to make sure the circuit is correct. As each technology library handles bi-directional ports differently, a check is made to determine whether the port is used as a bi-directional. If so, the correct netlist of pad cells is constructed and inserted.

An additional problem arises when there are multiple tri-states driving a single bi-directional output.  For example, several elements on an internal bus may be connected off-chip. In this case, one approach would be to map each tri-state into a different output pin. This approach has the drawback that multiple chip pins are used for a single signal. Instead, ViewSynthesis for Windows performs a more complex transformation which uses only a single signal.

ViewSynthesis for Windows transforms the circuit by inserting a single bi-directional pad, using a single chip pin.  The internal tri-states remain internal tri-states and are connected to the input of the bi-directional driver. The “ENABLE” signal for the bi-directional signal is determined so that whenever any of the internal tri-states is active, the pad tri-state is active as well. For active low drivers, the individual enables are ANDed; for active high drivers, they are ORed.

Sometimes a design uses more clocks then the number of clock buffers than the technology family supports. When this happens, you can specify a priority for each clock signal This will guide the tool to connect the special clock buffers to the highest priority clock signals. The rest will be connected through ordinary inputs.

You do this by creating a file in the primary directory <entity>.fpn.

Here is an example where the clock2.vhd VHDL file uses six clocks, clk[0] through clk[6].

Here is clock2.fpn:

| Tests to make sure clock priority setting works correctly
| For a four clock part, 0 and 1 must be there; two out of
| 2, 3, 4 will be there, and 5 had better not be there.
clock 1 clk[0]
clock 1 clk[1]
clock 2 clk[2]
clock 2 clk[3]
clock 2 clk[4]

### Clock Buffering

This section describes how clock buffers are inserted. Many FPGA technologies provide specific low skew clock routing channels on the chip. These routing channels are accessed by driving the net with a specific clock buffer. Rather than using a general purpose input buffer for these nets, ViewSynthesis for Windows uses the specific clock buffer, resulting in lower skew for the clock nets and higher performance.

When ViewSynthesis for Windows processes the design, it determines all of the input signals which are used as clock inputs to any flip-flop or latch in the design. When appropriate clock buffers are available in the technology library, these buffers are connected to the clock signals, replacing default input buffers.

### Default Pads

This section describes how default input and output pads are added to the design. In the previous sections, the initial processing steps are described; as a result, some, but not all, of the ports in the design will be correctly constructed to go into the FPGA place and route environment. The final processing step transforms the remaining ports of the design so that the entire design may be transferred to FPGA place and route.

Each FPGA technology provides a set of symbols which must be connected to input and output ports.  In some cases, only a single element is required. For some technologies, both a single pin “pad” symbol and a two pin “buffer” symbol must be attached. For each port which has not been modified by complex I/O insertion or clock buffer insertion, the default “pad” and/or “buffer” symbols are added. At the conclusion of this process, all ports of the design have been transformed into the representation required by the FPGA place and route process.

### Specifying I/O Pad Locations

An I/O signal can be assigned to a specific pad location by entering information in a file in the primary directory <entity>.fpn.

In this example, output g[0] is assigned pad location P3:

| Assign one pad location
buffat g[0] loc=P3;

Overview

Introduction

This chapter reviews the Xilinx technology specific optimization and mapping. It reviews the basics of “Look Up Tables” (LUTs), “Combinational Logic Block” (CLBs), and explains how to interpret Xilinx specific synthesis reports.

Xilinx families XC2000 and XC3000 are mapped into CLBs. Xilinx families XC4000 and XC4000h are mapped into LUTs. Other Xilinx families are not mapped into “normal” cells in the technology library.

Invoking Technology Specific Mapping

ViewSynthesis for Windows performs technology specific mapping, where applicable by default. This mapping can be turned off by using the Synthesis Options button, then click on the Use Technology Specific Mapping option.

CLB and LUT Basics

Xilinx architecture is made up of LUTs that form an FPGA design’s basic structure. (Refer to Xilinx technical documentation for detailed Xilinx architectural information.)

A LUT is essentially a 16-bit RAM with 4 address lines. The 4 address lines are used to look up values in the register’s table, which in turn define a function. A lookup table is also synonymously referred to as a “function generator.”

A LUT can perform any function that includes 4 inputs. So, an AND gate, or an XOR gate have the same cost in Xilinx architecture. In fact, a complex function that may include 20 or 30 gates but only has 4 inputs, still collapses into the same space.

Xilinx architecture includes three function generator types:

- F generator: 4-input

- G generator: 4-input

- H generator: 3-input

A combination of three LUTs as shown in Figure C-12 constitutes a CLB. A CLB has 2 outputs and 2 flip-flops.

Figure 4-1. A Combinational Logic Block (CLB)

The Report File

Unlike mask-level ASICs, The Xilinx FPGAs are much more sensitive to Place and Route issues. ViewSynthesis for Windows operates at a level that is a good compromise between complete placement, and ignoring placement altogether. As such, some of the standard report information is less useful for some Xilinx families than for others.

The most important general pieces of information from the ViewSynthesis for Windows report files are:

- Maximum delay

- Critical path delays

- Gate usage summary

### XC4000 Reports

For the XC4000 family, ViewSynthesis for Windows maps most logic down to LUTs. The major exception is that ViewSynthesis for Windows also targets X-BLOX modules for arithmetic operations.

Many  X-BLOX modules use special routing to speed up computations. Thus, a logic level count is not useful when X-BLOX modules are being inferred. However, ViewSynthesis for Windows can predict with fair precision the absolute delay for a given Xilinx speed grade part for inferred X-BLOX nodules. On the other hand, without PP&R, the predicted absolute delay information is substantially inaccurate. We thus recommend that you run Xilinx PP&R for delay information.

The area predictions based on LUTs can, however, be fairly accurate. Here is a “Gate Usage Summary” fragment of a ViewSynthesis for Windows report.

******************************************************

Gate Usage Summary

******************************************************

------------------------------------------------------------------

Cell				Count							Area/Cell

--------------------------------------------

FMAPS			HMAPS			REGISTERS

------------------------------------------------------------------

VW_I_16			1					14.50			0.00				 0.00

XC4000:FDCE	16					 0.00			0.00				 1.00

XC4000:FMAP	16					 1.00			0.00				 0.00

XC4000:HMAP	1					 0.00			1.00				 0.00

XC4000:IN		8					 0.00			0.00				 1.00

------------------------------------------------------------------

Total Cells	:				   34

Total Area	:					30.50			1.00				16.00

When synthesizing to LUTs, the important area statistics are:

- FMAPs

- HMAPs

- REGISTERs

Generally speaking these cannot be interchanged, so we keep them separate at this point for maximum information.

Notice here, a subcomponent VW_I_16 (an incrementer inferred  by ViewSynthesis for Windows) is referenced. ViewSynthesis for Windows has synthesized the incrementer already, probably using an X-BLOX module, and predicted what the X-BLOX module generator will produce for area; 14.5 FMAPs, no HMAPs, and no REGISTERs. A fractional FMAP usually means that only two (of four) inputs to an FMAP have been used up, and opportunities for merging further logic into the FMAP may occur.

### XC2000 and XC3000 Reports

For these earlier Xilinx families, ViewSynthesis for Windows goes a bit further and maps LUTs and CLBs. The ViewSynthesis for Windows area report for these technologies looks like this:

******************************************************

Function Generator Usage

******************************************************

Total	flipflops	:		11

Total	CLBs			:		19

Overview

Full Compatibility with Xilinx Tools

ViewSynthesis for Windows can include timing constraints in its synthesis wirefile in Xilinx XACT tool format.

Including Timing Data in WIR File

ViewSynthesis for Windows contains timing constraints in its synthesis wirefile. In the Synthesis options dialog box, click on the Tech Options button, then click on the XACT Performance option in the Xilinx options dialog box.

Overview

Introduction

All of the built-in VHDL arithmetic operators (+, -, <, >, <=, >=, =, /=) infer X-BLOX or equivalent module generators automatically. This appendix describes a way to instantiate X-BLOX explicitly, as opposed to being automatically inferred from behavioral VHDL.

The X-BLOX VHDL Package

The X-BLOX VHDL package listed later in this section is supplied with ViewSynthesis for Windows as xblox.vhd. It is presented here for your information only. This package is installed along with the $WDIR path during the installation of ViewSynthesis for Windows and must be pointed to in the viewdraw.ini file and declared at the beginning of a VHDL description that uses X-BLOX procedure calls. An example of its use is presented in Chapter 1’s “Design Examples” section.

WARNING:

Do not modify this file. Changes to the X-BLOX VHDL package may adversely effect synthesis results.

### Passing Inputs, Outputs and Attributes to X-BLOX

Use the X-BLOX VHDL package listing as a reference when assigning X-BLOX inputs, outputs, and attributes. Inputs, outputs, and attributes can be assigned in one of two ways:

- Using a named concurrent procedure call

- Using a positional concurrent procedure call

IMPORTANT:

X-BLOX procedure calls must be concurrent. They cannot be invoked inside processes.

The following two examples show how to use a named and a positional procedure call. Both examples get basic X-BLOX information from the following xb_COUNTER procedure excerpted from the X-BLOX VHDL Package. Both examples use this information to define the COUNTER portion of the Counter/BCD to 7-Segment assembly described in Chapter 1, “Designing with Xilinx X-BLOX”.

procedure xb_COUNTER(
        d_in: in bit_vector;
        load, up_dn, async_ctrl, sync_ctrl, clk_en, clock: in bit;
        q_out: out bit_vector;
        term_cnt: out bit;
        attr: in string );
                -- ASYNC_VAL= 0
                -- SYNC_VAL= 0
                -- STYLE= BINARY/JOHNSON/LFSR
                -- COUNT_TO= 10
                --
                --

Figure A-1. xb_COUNTER Procedure

### Example:  Using a Named Association Procedure Call

In a named association procedure call there is no relationship between the X-BLOX VHDL Package listing order and the order that you make X-BLOX assignments.

This excerpt from the Counter/BCD to 7-Segment description shows how Formal names and Actual parameters are assigned to each other in a named association relationship.

Formal				Actual

xb_COUNTER(ASYNC_CTRL	=> RESET,
										SYNC_CTRL		=> COUT,
										CLOCK			=> CLK,
										CLK_EN			=> DN_ENABLE,
										Q_OUT			=> BCD,
										ATTR				=> ”STYLE=BINARY”);

|		  |

Figure A-2. Named Association Procedure Call

### Example: Using a Positional Procedure Call

To use a positional procedure call, list the X-BLOX input, output, and attribute Actuals in the exact top-down order as they appear in the xb_COUNTER procedure in the X-BLOX VHDL Package listing.

The following positional procedure call passes the identical inputs, outputs and attributes to X-BLOX as the named association example. Actual parameters are listed in exactly the same order as they appear in the xb_COUNTER procedure. When an input or output is not used, the word “open” acts as a placeholder for that Actual. For example, in Figure C-5, pins D_IN, LOAD, UP_DN and TERM_CNT are not used and have no related Actual in the design. Therefore, the word “OPEN” is used to hold their places.

xb_COUNTER(OPEN,OPEN,OPEN,RESET,COUT,
CLK,DN_ENABLE,BCD,OPEN,”STYLE=BINARY”);

Figure A-3. Positional Association Procedure Call

### Attribute Syntax and Usage

Attribute(s) must be enclosed in quotation marks as shown here:

ATTR => ”STYLE=BINARY”

To pass more than one attribute to X-BLOX, separate each attribute with a reverse single quote ( ‘ ) as shown here:

ATTR => ”STYLE=BINARY‘COUNT_TO=9”

Note:

The attribute string is space sensitive.

### X-BLOX VHDL Package Listing

package xblox is
  procedure xb_ACCUM(
        add_sub: in bit;
        c_in: in bit;
        b: in bit_vector;
        load, clk_en, clock, async_ctrl, sync_ctrl: in bit;
        q_out: out bit_vector;
        c_out, ovfl: out bit;
        attr: in string );
                -- ASYNC_VAL= 0
                -- SYNC_VAL= 0
                -- STYLE= HM/ALIGNED
                --
                --

procedure xb_ADD_SUB(
        add_sub: in bit;
        c_in: in bit;
        a: in bit_vector;
        b: in bit_vector;
        func: out bit_vector;
        c_out: out bit;
        ovfl: out bit;
        attr: in string );
                -- STYLE= HM/ALIGNED
                --
                --

procedure xb_BIDIR_IO(
        oe: in bit;
        outputs: in bit_vector;
        ie: in bit;
        inputs: out bit_vector;
        attr: in string );
                -- ENCODING= BIT/UBIN/TWO_COMP/ONE_HOT
                -- BOUNDS= 0:7/7:0
                -- FLOAT_VAL= 1/0/?/PULLUP/PULLDOWN
                -- LOC=A1
                -- LOC[6]= A11 ‘LOC[7]= P9
                --
                --

procedure xb_BUS_DEF(
        xblox_bus: in bit_vector;
        attr: in string );
                -- ENCODING= BIT/UBIN/TWO_COMP/ONE_HOT
                -- BOUNDS= 0:7/7:0
                --
                --

procedure xb_CLK_DIV(
        async_ctrl, sync_ctrl, en, clock: in bit;
        clk_out: out bit;
        attr: in string );
                -- DIVIDE_BY= 3
                -- DUTY_CYCLE= 1
                --
                --

procedure xb_COMPARE(
        a: in bit_vector;
        b: in bit_vector;
        a_eq_b, a_ne_b, a_lt_b, a_gt_b, a_le_b, a_ge_b: out bit;
        attr: in string );
                -- STYLE= ARITH/ARITHWIRED/RIPPLE/WIRED/TREE
                --
                --

procedure xb_COUNTER(
        d_in: in bit_vector;
        load, up_dn, async_ctrl, sync_ctrl, clk_en, clock: in bit;
        q_out: out bit_vector;
        term_cnt: out bit;
        attr: in string );
                -- ASYNC_VAL= 0
                -- SYNC_VAL= 0
                -- STYLE= BINARY/JOHNSON/LFSR
                -- COUNT_TO= 10
                --
                --

procedure xb_DATA_REG(
        d_in: in bit_vector;
        async_ctrl, sync_ctrl, clk_en, clock: in bit;
        q_out: out bit_vector;
        attr: in string );
                -- ASYNC_VAL= 0
                -- SYNC_VAL= 0
                --
                --

procedure xb_DECODE(
        sel: in bit_vector;
        en: in bit;
        d_out: out bit_vector;
        sel_error: out bit;
        attr: in string );
                --
                --

procedure xb_ELEMENT(
        xblox_bus: in bit_vector;
        elem: in bit;
        attr: in string );
                -- ELEM= 3
                --
                --

procedure xb_FORCE(
        force: out bit_vector;
        attr: in string );
                -- VALUE= 2#00001111#
                --
                --

procedure xb_INC_DEC(
        inc_dec: in bit;
        a: in bit_vector;
        func: out bit_vector;
        c_out: out bit;
        attr: in string );
                -- STYLE= HM/ALIGNED
                --
                --

procedure xb_INPUTS(
        inputs: out bit_vector;
        attr: in string );
                -- ENCODING= BIT/UBIN/TWO_COMP/ONE_HOT
                -- BOUNDS= 0:7/7:0
                -- FLOAT_VAL= 1/0/?/PULLUP/PULLDOWN
                -- LOC=A1
                -- LOC[6]= A11 ‘LOC[7]= P9
                --
                --

procedure xb_MUXBUS(
        mux_in: in bit_vector;
        sel: in bit_vector;
        mux_out: out bit_vector;
        sel_error: out bit;
        attr: in string );
                --
                --

procedure xb_MUXBUS2(
        m1: in bit_vector;
        m0: in bit_vector;
        sel: in bit_vector;
        mux_out: out bit_vector;
        sel_error: out bit;
        attr: in string );
                --
                --

procedure xb_MUXBUS4(
        m3: in bit_vector;
        m2: in bit_vector;
        m1: in bit_vector;
        m0: in bit_vector;
        sel: in bit_vector;
        mux_out: out bit_vector;
        sel_error: out bit;
        attr: in string );
                --
                --

procedure xb_MUXBUS8(
        m7: in bit_vector;
        m6: in bit_vector;
        m5: in bit_vector;
        m4: in bit_vector;
        m3: in bit_vector;
        m2: in bit_vector;
        m1: in bit_vector;
        m0: in bit_vector;
        sel: in bit_vector;
        mux_out: out bit_vector;
        sel_error: out bit;
        attr: in string );
                --
                --

procedure xb_OUTPUTS(
        outputs: in bit_vector;
        attr: in string );
                -- ENCODING= BIT/UBIN/TWO_COMP/ONE_HOT
                -- BOUNDS= 0:7/7:0
                -- FLOAT_VAL= 1/0/?/PULLUP/PULLDOWN
                -- LOC=A1
                -- LOC[6]= A11 ‘LOC[7]= P9
                --
                --

procedure xb_PROM(
        addr: in bit_vector;
        d_out: out bit_vector;
        addr_error: out bit;
        attr: in string );
                -- MEMFILE= filename
                -- DEPTH= 1023
                --
                --

procedure xb_SHIFT(
        par_in: in bit_vector;
        load, ms_in, ls_in, right_left: in bit;
        async_ctrl, sync_ctrl, clk_en, clock: in bit; 
        par_out: out bit_vector;
        ls_out, ms_out: out bit;
        attr: in string );
                -- STYLE= LOGICAL/CIRCULAR/ARITH
                -- ASYNC_VAL= 0
                -- SYNC_VAL= 0
                --
                --

procedure xb_SLICE(
        main: in bit_vector;
        sub: in bit_vector;
        attr: in string );
                -- SLICE= 3:0
                -- SUB_STARTS_AT= 4
                -- SUB= 3:0
                --
                --

procedure xb_SRAM(
        d_in: in bit_vector;
        wr_en: in bit;
        addr: in bit_vector;
        d_out: out bit_vector;
        addr_error: out bit;
        attr: in string );
                -- DEPTH= 777
                --
                --

procedure xb_TRISTATE(
        oe: in bit;
        i: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- FLOAT_VAL= 1/0/?/PULLUP/PULLDOWN
                --
                --

-----
  procedure xb_ANDBUS(
        a: in bit_vector;
        o: out bit;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_ANDBUS1(
        a: in bit;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_ANDBUS2(
        a: in bit_vector;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

-----
  procedure xb_INVBUS(
        i: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

-----
  procedure xb_ORBUS(
        a: in bit_vector;
        o: out bit;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_ORBUS1(
        a: in bit;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_ORBUS2(
        a: in bit_vector;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

-----
  procedure xb_XORBUS(
        a: in bit_vector;
        o: out bit;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_XORBUS1(
        a: in bit;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_XORBUS2(
        a: in bit_vector;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_XORBUS2(
        a: in bit_vector;
        b: in bit_vector;
        o: out bit_vector;
        attr: in string );
                -- INVMASK= 2#1010#
                --
                --

procedure xb_OUTELEM(
        xblox_bus: in bit_vector;
        elem: out bit;
        attr: in string );
                -- ELEM= 2
                --
                --
end xblox;