addtnm.doc

Revision 2.2

----------------------------------------------------------------------------

DISCLAIMER

addtnm and maketnm are not official Xilinx software. Please report any

problems or suggestions to Randy Saldinger at randy@xilinx.com.

----------------------------------------------------------------------------

INSTALLATION

addtnm and maketnm are PERL scripts. If PERL is not located at

/usr/local/bin on your system, it will be necessary to modify the

first line of each script to point to the perl executable.

Alternatively, you can simply invoke PERL directly as follows:

perl addtnm <the usual arguments>

addtnm and maketnm were written and tested with PERL V4.0.

----------------------------------------------------------------------------

INTRODUCTION

The addtnm script reads an XFF file and a control file, which defines

the group names that should be applied to the design.

There are two ways to make a control file:

(1) Use the maketnm script to make a template control file, and edit

that file to add group names. This is described below.

(2) If the instances names are already known, create the control file

from scratch, using the syntax explained in the next section.

----------------------------------------------------------------------------

CONTROL FILE SYNTAX

The control file should have the same base name as the .xff file, and

should carry a .tnm extension. The syntax of the .tnm file is similar

to the PPR CST file:

<endpoint-type> <instance-name> : <group> [ <group2> ... ]

where:

<endpoint-type> is one of FFS, RAMS or PADS. IOB latches are lumped

together with the FFS type by addtnm, as are XBLOX

modules which will expand to include flip-flops.

<instance-name> is the name of the symbol(s) which should be tagged

with the provided group names. Like CST file placement

constraints, the wildcards ? (single character) and *

(multiple character) may be used to specify multiple

symbols. Only symbols of the type given by <endpoint-type>

will be considered.

<group> is the name to assign to the specified instance(s). This name

can then be used as endpoints in FROM-TO TIMESPEC statements,

or as building blocks in TIMEGRP statements (both of which can

be specified via the CST file). Multiple names can be assigned

to an instance; separate group names by spaces or tabs.

As with the CST file, lines that being with a pound (#) character are

treated as comments. UNLIKE the CST file, comments cannot be included

on the same line as a statement. Also unlike the CST file, there is no

semicolon at the end of a statement.

In addition to applying group names to instances, addtnm can also be

used to apply group names to signals. This is an easy way to group flip-

flops that have a common clock or enable signal. (See page 1-76 in Volume

I of the XACT Reference Guide for more information on this.) To do so,

add a line of the following form to the .tnm control file:

SIG <signal-name> : <group> [ <group2> ... ]

where <signal-name> is the name of the signal(s) to tag. As with the

instance-based statements, the * and ? wildcards can be used in signal

names. These statements can be applied to input signals -- such as

clock inputs -- to tag the flip-flops they control.

An example .tnm file is shown below:

# example design.tnm file

#

FFS REG_3A/* : reg_3a_group

FFS REG_3?/Q0 : bit0_group

FFS REG_3C/Q7 : 3c7_group other_group

PADS ENA : enable_input

SIG CLOCK40 : mainclk

All items in the .tnm file are case-insensitive.

----------------------------------------------------------------------------

USING maketnm TO CREATE A TEMPLATE CONTROL FILE

The maketnm script reads an XFF file and outputs a template control file.

Invoke maketnm as follows:

maketnm <design>

The template file will be written to <design>.tt. It lists all of the

flip-flops, RAMs and I/O pads in the design, by instance name. XBLOX

modules which expand to include flip-flops are also listed, prefaced

by the FFS keyword.

To create a .tnm file, add the desired group names to the appropriate

instances. Instance names can be modified to use wildcards, as detailed in

the preceeding section. If no group names are added to a statement, that

statement will be ignored by addtnm.

After modifying the template file, change the name to <design>.tnm, so

that addtnm can find it, as discussed below.

----------------------------------------------------------------------------

USING addtnm TO MERGE THE CONTROL FILE INFORMATION

Once a .tnm control file has been created, it can be used by the addtnm

script each time the design is processed. addtnm is designed to be run

on a flattened XFF file. As such, it should be run between the XNFMERGE

and XNFPREP programs.

addtnm is invoked as follows:

addtnm <design>

The XFF file should exist as <design>.xff and the control file should

exist as <design>.tnm. To avoid overwriting the source XFF file, the

modified XFF file is written to <design>.txff. This can be renamed back

to a .xff extension before running XNFPREP.

For example, a script to process the design might include the following

steps:

xnfmerge <design>

addtnm <design>

mv <design>.txff <design>.xff

xnfprep <design>

To examine exactly which group names are being assigned, addtnm provides

a debug mode which details each group name assigned to each instance. To

invoke addtnm with this verbose output, use:

addtnm -d <design>

---------------------------------------------------------------------------

XACT-Performance: Specifying TIMEGRP and TIMESPEC in CST File

As an alternative to the TIMESPEC and TIMEGRP schematic symbols,

specifications and group definitions may be placed in the CST file.

However, the documentation is somewhat unclear on how to do this.

The CST file can be used to specify parameters that would otherwise

be attached to a TIMESPEC or TIMEGRP symbol. (It is not possible to

use the CST file specify TNM parameters that would otherwise be

attached to symbols or signals.) The basic syntax is as follows:

TIMESPEC = "<timespec-parameter>" ;

TIMEGRP  = "<timegrp-parameter>" ;

The <timespec-parameter> is the same text that would be attached

to a TIMESPEC symbol. The <timegrp-parameter> is the same text

that would be attached to a TIMEGRP symbol. (See the "XACT-Performance

Utility" chapter in Volume I of the XACT Reference Guide for more

information on these parameters.) The spaces in the above syntax are

optional; the double quote marks around the parameter text are required.

Several examples of TIMESPEC and TIMEGRP CST file statements are

shown below.

TIMEGRP  = "updata_io=PADS(updata<*>)" ;

TIMESPEC = "TS04=FROM:LATCHES:TO:updata_io=50ns" ;

TIMEGRP  = "CLK40_POS=RISING:CLK40" ;

TIMEGRP  = "CLK40_NEG=FALLING:CLK40" ;

TIMESPEC = "TS02=FROM:CLK40_POS:TO:CLK40_POS=20MHz" ;

TIMESPEC = "TS03=FROM:CLK40_POS:TO:CLK40_NEG=40MHz" ;